Understanding End Gas Auto-Ignition in n-Heptane Combustion at High Temperatures using 2-D DNS

A PROJECT REPORT

Submitted in partial fulfilment of the

requirements for the award of the degrees

of

BACHELOR OF TECHNOLOGY

in

MECHANICAL ENGINEERING

Submitted by:

Arjav Jain		(19117017)

Shreyam Bansal	(19119038)

Guided by:

Prof. Ajit Kumar Dubey

INDIAN INSTITUTE OF TECHNOLOGY ROORKEE

May 2023

B. TECH. PROJECT REPORT

On

Understanding End Gas Auto-Ignition in n-Heptane Combustion at High Temperatures using 2-D DNS

BY

Arjav Jain		(19117017)

Shreyam Bansal	(19119038)

Department of Mechanical and Industrial Engineering

INDIAN INSTITUTE OF TECHNOLOGY ROORKEE

May 2023

CANDIDATE’S DECLARATION

We hereby declare that the project entitled “Understanding End Gas Auto-Ignition in n-Heptane Combustion at High Temperatures using 2-D DNS”, submitted in partial fulfilment for the award of the degree of Bachelor of Technology in ‘Mechanical Engineering’ completed under the supervision of Prof. Ajit Kumar Dubey, IIT Roorkee is an authentic work.

Further, we declare that we have not submitted this work for the award of any other degree elsewhere.

Date: 5th May 2023

Signature			      Signature

Arjav Jain (19117017)	Shreyam Bansal (19119038)

_________________________________________________________________

CERTIFICATE by BTP Guide

It is certified that the above statement made by the students is correct to the best of my knowledge.

Signature

Prof. Ajit Kumar Dubey

Assistant Professor

Department of Mechanical and Industrial Engineering

IIT Roorkee

Acknowledgements

First of all, the team wishes to place on record its appreciation and gratitude to the Indian Institute of Technology Roorkee, which gave us this excellent opportunity of the course B. Tech Project and provided various resources during the course of the project.

We wish to thank Prof. Ajit Kumar Dubey for his kind support and valuable guidance. It was his help and support that we were able to complete the design and technical report. Without his support, this report would not have been possible.

This project gave us the opportunity to apply the concepts discussed in the classrooms to the real world and, in turn, enabled us to understand the concepts of mechanical engineering even better.

Arjav Jain

Shreyam Bansal

B.Tech. IV Year

Department of Mechanical & Industrial Engineering

IIT Roorkee

Abstract

The main objective of the project is to identify the specific locations and mechanisms of auto-ignition during the combustion of n-heptane and gain insights into how temperature and pressure impact this process. To achieve this goal, the project will use Computational Fluid Dynamics (CFD) to conduct simulations and analyze the resulting data. The study will focus on 1D and 2D models of pressure wave behavior within an n-heptane combustion engine and incorporate detailed chemistry models to better understand the different auto-ignition modes that occur during combustion. By investigating the interactions between pressure waves and hot spots, and how these factors contribute to knocking and detonation, the project aims to provide a more precise understanding of the effects of pressure waves under various initial condition	s.

Table of Contents

| S. No. | List of Contents | Page No. |
| --- | --- | --- |

|  | Candidate’s Declaration | III |
| --- | --- | --- |
|  | Acknowledgement | IV |
|  | Abstract | V |
|  | Table of Contents | VI |
|  | List of Figures | VII |

| Chapter 1 | Introduction | 1 |
| --- | --- | --- |
| 1.1 | Internal Combustion Engine | 1 |
| 1.2 | Premixed and Diffusion Flames | 2 |
| 1.3 | Spark Ignition | 3 |
| 1.4 | Knocking | 4 |

| Chapter-2 | Project Motivation | 6 |
| --- | --- | --- |

| Chapter-3 | Problem Definition | 7 |
| --- | --- | --- |

| Chapter-4 | Literature Review | 8 |
| --- | --- | --- |

| Chapter-5 | Methodology and Numerical Models | 10 |
| --- | --- | --- |
| 5.1 | Governing Equations | 10 |
| 5.2 | Numerical Model | 11 |
| 5.3 | Computational Model | 11 |
| 5.4 | Boundary Conditions | 13 |
| 5.5 | Grid Convergence Study | 14 |

| Chapter-6 | Results and Discussion | 18 |
| --- | --- | --- |
| 6.1 | Autoignition Modes | 18 |
| 6.2 | Autoignition Modes at 3 atm and 1150 K | 19 |
| 6.2.1 | Numerical Estimation of Laminar Flame Speed | 28 |
| 6.2.2 | Numerical Estimation of C-J Detonation Speed | 28 |
| 6.2.3 | Analytical Estimation of Laminar Flame Speed | 29 |
| 6.2.4 | Quantification in Bradley’s Diagram | 34 |
| 6.3 | Autoignition Modes at High-Pressure Conditions | 36 |

|  | Conclusion | 40 |
| --- | --- | --- |

|  | References | 41 |
| --- | --- | --- |

|  | Appendix | 45 |
| --- | --- | --- |

List of Figures and Tables

Figures

Figure 1.1:   	Location of an ICE in a Commercial Automobile

Figure 1.2:	Premixed Flames

Figure 1.3:	Turbulent Diffusion Flames

Figure 1.4:	Components of a SI Engine

Figure 1.5:	Cylinder pressure versus crank-angle traces of individual cycles with (a) normal combustion, (b) light knock, and (c) heavy knock.

Figure 2.1:	Damage due to Conventional Knock (left) and Super-Knock (right)

Figure 3.1:	Schematic Depiction of Auto-Ignition Modes at Different Initial Temperatures

Figure 5.1:	Schematic of the Computational Domain

Figure 5.2:	Schematic of the One- and  Two- Dimensional Computational Domains

Figure 5.3:	Pressure History at P1 using

Figure 5.4:	Pressure Histories at P1 and P2

Figure 5.5:	dP/dt Histories at P1

Figure 6.1:	Scenarios of  Local Auto-Ignition and Flame Propagation at Different Initial Temperatures

Figure 6.2:	Temperature Contours at Different Timesteps

Figure 6.3:	Pressure Contours at Different Timesteps

Figure 6.4:	Temperature and Pressure Plots about Centreline at Different Timesteps @ 3atm 1150K

Figure 6.5:	Temperature Distribution at Various Instances Clarifying the Auto-Ignition Mode

Figure 6.6:	Plots of Temperature and Pressure at Important Timesteps

Figure 6.7:	Temperature Plots (left) and Pressure Plots (right) at Certain Timesteps

Figure 6.8:	Rate of Change of Pressure (dP/dt) and Temperature (dT/dt) w.r.t Y-axis

Figure 6.9:	Temperature Plots (left) and Pressure plots (right) @ X=7.985

Figure 6.10:	Rate of Change of Pressure (dP/dt) and Temperature (dT/dt) w.r.t. Y-axis @ X=7.985

Figure 6.11:	Temperature Variation near the Flame Front (Zooming the peak as we move down)

Figure 6.12:	OH Mass Fraction Variation near the Flame Front (Zooming the peak as we move down)

Figure 6.13:	AI Reaction Front Speed for End-Wall AI with Sound Speed and C-J Detonation Speed

Figure 6.14:	Comparison of AI Zones based on Temperature and Mass Fraction of OH

Figure 6.15:	Summary of AI Mode and Knocking Intensity in Non-Dimensional Bradley’s Diagram

Figure 6.16:	Pressure Contours at Different Instances @ 10atm 1150K

Figure 6.17:	Temperature Contours at Different Instances @ 10atm 1150K

Figure 6.18:	Temperature and Pressure Plots about Centerline at Different Timesteps @10atm 1150K

Chapter 1

Introduction

  - Internal Combustion Engines

An internal combustion engine (ICE) is a type of heat engine that uses a combustion chamber to burn a fuel with an oxidizer, typically air, within a working fluid flow circuit. This combustion produces high-temperature and high-pressure gases that generate force on components of the engine, such as pistons, turbine blades, rotors, or nozzles. This force is converted into kinetic energy, which propels the engine and its attached equipment. ICEs can be divided into two categories: intermittent combustion engines, such as two-stroke and four-stroke piston engines and the Wankel rotary engine, and continuous combustion engines, such as gas turbines, jet engines, and most rocket engines.

Although ICEs have a wide range of stationary applications, they are primarily used to power mobile equipment such as cars, boats, and aircraft. These engines typically run on hydrocarbon-based fuels such as natural gas, gasoline, diesel, or ethanol, while renewable fuels like biodiesel and bioethanol are used in compression ignition (CI) and spark ignition (SI) engines, respectively. Internal combustion engines can be produced in various sizes and can be installed in small devices as well. They are considered safe as they don't pose a significant risk of accidents. Another advantage of internal combustion engines is their ability to start instantly.

Figure 1.1: Location of an ICE in a Commercial Automobile

  - Premixed and Diffusion Flames

Chemical reactions in a flame transform fuel vapours into burnt gases, producing carbon dioxide and water vapour. Though it requires some heat energy to begin, the reaction generates more heat than it consumes, ensuring self-sustaining burning.

Premixed flames are formed by thoroughly mixing fuel with an oxidant, such as air. To ignite, a spark or small flame is required. Once ignited, the flame will become self-sustaining and propagate in all directions. In the flame zone, cold reactants are transformed into hot products. The thickness of the premixed flame zone is typically less than 1 mm. Due to the greater volume of hot burnt gas compared to cold unburnt gas, the flame front expands outward from the ignition point, similar to the skin of an inflating balloon.

In Figure 1.2, different types of flames are discussed. A yellow diffusion flame, which is sooty, is produced by a fuel-rich mixture with no premixed oxygen on the leftmost. On the rightmost, a lean premixed flame with fully oxygen produces no soot, and the color of the flame comes from molecular radical band emission.

Figure 1.2 Different flame types of a Bunsen burner depend on oxygen supply.

The interface between fuel vapour and air is where diffusion flames are formed, in contrast to premixed flames where fuel and oxidant are already mixed. In diffusion flames, mixing is the main process. Molecular diffusion causes the fuel vapour and oxygen to mix, which is a slow process, but the high temperatures present in flames make it happen more rapidly.

Diffusion flames don't have a burning velocity, and there are no equivalents to flammability limits, rich or lean mixtures because they exist solely at the fuel-air interface.

Diffusion flames fall into two broad types.

- Laminar diffusion flame – Slow-burning diffusion flames, like those in candles, are characterized by a gradual rise of the fuel vapour in a laminar flow dominated by molecular diffusion from the wick.

- Turbulentadiffusionaflamea-a- Industrialaburnersacreateaturbulentadiffusionaflamesabyainjectingafuelaintoatheaairaatahighavelocity,aeitheraasaaasprayaoraaajet.aTurbulenceaisainducedaatatheainterfaceawhereatheafuelaandaairamix,acreatingaanaextensiveasurfaceaareaaforatheaflameacomparedatoatheasmoothafuel/airainterfaceaofaaacandleaflame.aInathisacase,atheamixingarateaisadeterminedabyathealargeainterfaceaarea,aratherathanatheamolecularadiffusionarate.

Figure 1.3: Turbulent Diffusion Flames

  - Spark Ignition

An SI engine (Spark Engine) is a type of internal combustion engine that uses a spark from a spark plug to ignite the air-fuel mixture, usually consisting of gasoline. This differs from diesel engines, which are compression-ignition engines, where combustion is initiated through the heat generated by the compression of air and the injection of fuel, without the need for a spark.

Figure 1.4: Components of a SI Engine

Bothaspark-ignitionaandacompression-ignitionaenginesacanaoperateawithaeitheraaatwo-strokeaorafour-strokeaworkingacycle.aAafourstrokeasparkignitionaengineafollowsatheaOttoacycleaandaincludesafourastrokes,anamelyatheaintakeaorasuctionastroke,acompressionastroke,apoweraoraexpansionastroke,aandaexhaustastroke.aTheacrankshaftarotatesa180adegreesaforaeachastroke,aleadingatoaaacompleteafourstrokea cycleaachievedaovera720adegreesaofacrankshaftarotation.aAsaaaresult,aeachacycleaincludesaonlyaoneapowerastroke,awhileatheacrankshaftarevolvesatwoaoramoreatimes.

  - Knocking

Knocking,aalsoareferredatoaasadetonation,asparkaknock,apinging,aorapinking,aisaaaphenomenonathatacanaoccurainasparkaignitionainternalacombustionaengines.aThisaoccursawhenatheacombustionaofasomeaofatheaair/fuelamixtureainatheacylinderadoesn’tapropagateafromatheaflameafrontaignitedabyatheasparkaplug.aInstead,aoneaoramoreapocketsaofaair/fuelamixtureaexplodeaoutsideatheaenvelopeaofathearegularacombustionafront.aThisacanahappenawhenatheaair/fuelachargeaisaignitedaataaapointainatheapiston'sastrokeathataisanotatheaoptimumamomentaforatheafourstrokeacycle.aTheashockawaveageneratedabyatheaexplosionacreatesaaadistinctiveametallica"pinging"asound,aandacylinderapressureaincreasesasignificantly

The effects of engine knocking can range from inconsequential to completely destructive, causing engine damage in some cases. The severity of knocking depends on various factors, including engine design, air/fuel mixture composition, engine operating conditions, and more. Engine knocking can cause the engine to run rough, reduce power output, increase fuel consumption, and in severe cases, cause piston or other engine component damage. Engine designers use various techniques to minimize the likelihood of knocking, such as designing the engine for optimal combustion timing, using high octane fuel, and modifying the shape of the combustion chamber to promote better mixing of the air/fuel mixture.

Detonationaisaanaexplosiveaphenomenonathataoccursawhenaanaunburnedafuel/airamixture,alocatedabeyondatheaboundaryaofatheaflameafront,aisaexposedatoaheataandapressureaforaaacertainadurationabeyondatheafuel'sadelayaperiod.aThisarapidacombustionaprocessacausesaanaalmostainstantaneousaignitionaofaataleastaoneapocketaofafuel/airamixtureaoutsideaofatheaflameafront.aTheadetonationawaveapropagatesathroughoutatheacombustionachamber,acreatingaaalocalashockwaveaaroundaeachapocketaandacausingaaasharpariseainacylinderapressureathatamayaexceedatheaengine'sadesignalimits,aresultingainadamage.aAlthoughadetonationaisamoreaefficientathanadeflagration,aitaisausuallyaavoidedadueatoaitsadetrimentalaeffectsaonaengineacomponents.

Figure 1.5 Cylinder pressure versus crank-angle traces of individual cycles with (a) normal combustion, (b) light knock, and (c) heavy knock.

DetonationainaanainternalacombustionaengineaoccursawhenaanaunburnedaF/Aamixtureabeyondatheaflameafrontaisasubjectedatoabothaheataandapressureaforaaadurationabeyondatheafuel'sadelayaperiod.aThisasituationamayaleadatoatheanearinstantaneousaandaexplosiveaignitionaofaataleasta1apocketaofaF/Aamixtureaoutsideaofatheaflameafront,awhichacreatesaaalocalashockwave.aTheasharpariseainacylinderapressureamayacauseadamageabeyondatheaengine'sadesignalimits.aAlthoughadetonationaisamoreaefficientathanadeflagration,aitaisausuallyaavoidedadueatoatheadamagingaeffectsaitahasaonaengineacomponents.aTherefore,aengineaknockacontrolasystemsaareadesignedatoaminimizeatheariskaofadetonationaandapreventadamageatoatheaengine.

Chapter 2

Project Motivation

As we know, knocking affects the engine’s life, causing damage to the piston-cylinder system, camshafts, and other mechanical components. The figure shows the different damages caused to the engine due to Conventional and Super-knock.

Figure 2.1: Damage due to Conventional Knock (left) and Super-Knock (right)

To design a robust engine design or an engine fuel with less knocking tendency, it is crucial to understand the auto-ignition zones inside a piston-cylinder system, which is the primary reason for knocking.

Over the years, extensive research has been performed to understand the phenomenon of engine knocking, involving a combination of experimental, numerical, and theoretical analyses. Withatheadevelopmentaofahighperformanceacomputersaandaadvancedanumericalamethods,acomputationalafluidadynamicsa(CFD)aanalysesawithadetailedachemicalakineticsahaveagainedaattention.aCFDasimulationsaprovideainformationaonatheacomplexacouplingabetweenafluidadynamicsaandachemicalareactionsathataisadifficultatoaobtainathroughaexperimentsaalone.aWhileaonedimensionalasimulationsaareainsightful,atheyaonlyapartiallyareproduceatheanaturalaprocessaofaknockaonsetainasparkaignitiona(SI)aengines,asuchaasatheadevelopmentaofatheaflameashapeaduringapropagationaandainteractionsabetweenatheaflameafrontaandageneratedapressureawaves.aToaovercomeathesealimitations,atwodimensionalasimulationsausingadetailedachemicalamodelsaforarealisticafuelsahaveabeenaproposedatoaunderstandatheaeffectsaofapressureawavesaonatheaentireaprocessaofaflameapropagation,aknockaonset,aandadetonationapropagation.aThisaapproachaprovidesaaamoreacomprehensiveaunderstandingaofatheaknockingaphenomenonainaSIaenginesaandaoffersaaapromisingatoolaforaengineadesignaandaoptimization.

Chapter 3

Problem Description

The project aims to understand the behaviour of pressure waves due to the end gas auto-ignition flame front inside an n-heptane-fueled constant volume chamber under different conditions. A 2-D simulation setup is made to simulate the results. The initial pressure and temperature conditions are varied to understand the phenomenon under different conditions. These modes are conceptually shown in Fig. 3.1, where arrows indicate the direction ofaflameafrontapropagationaand auto-ignition (Auto-Ignition) front propagation. At 1100 K, Auto-Ignition is observed in the end gas while the spark-ignited flame front has touched the side walls and propagates as a curved flame. At 1150 K, two distinct Auto-Ignition zones are observed; one slightly aheadaofatheaSI flame frontaand one inatheaendagas. aAt 1200 K, atheaAuto-Ignition zone is observed only ahead of the flame front.

Figure 3.1: Schematic Depiction of Auto-Ignition Modes at Different Initial Temperatures

DNS calculates the required parameters like temperature, pressure, mass fraction of all the species and density. Different contours and graphs of these parameters will help us understand the auto-ignition zones.

Chapter 4

Literature Review

Overatheayears,athereahaveabeenaextensiveastudiesaonaautoignitionamode,aprimarilyadueatoaitsarelevanceatoaknockingaintensitya[1atoa4].aZel'dovicha[5]awasaoneaofatheapioneersainathisafieldaandashowedathatadueatoatheanonuniformareactivityaofareactiveaflows,adifferentaautoignitionamodesaexist,aincludingasubsonicadeflagration,adetonation,aandasupersonicadeflagration.aBradleyaandahisacolleaguesa[6,a7]proposedaanaoperationalapeninsulaabasedaonaaasingleahotaspotawithaaaspecificatemperatureagradient,aconsistingaofaaanormalizedatemperatureagradienta(ξ)aandaaaratioaofaacousticatimeatoaexcitationatimeaofachemicalaenergyareleasea(ε).aThisapeninsulaahasabeenawidelyaadoptedatoaclassifyadifferentacombustionaregimesawithacleararegimeaboundariesainatermsaofaauto-ignitionamodeaduringaknockingacombustiona[8,a9].aChenaandahisacolleaguesa[10]alateraextendedatheaautoignitionatheoryatoatheanegativeatemperatureacoefficienta(NTC)aregionaofalargeahydrocarbonafuelsandafoundathataNTCachemistryadoesanotasignificantlyaaffectatheaautoignitionamodeaforafixedaξaandaε.

MorearecentaresearchabyaZhangaetaal.a[11]ahasashownathatabothathermalaandaconcentrationastratifications,aasawellaasatheiracross-coupling,acanasignificantlyaaffectatheadevelopmentaofaauto-ignitionamode.aHowever,ainamostapreviousaworks,aspecificathermalaoraconcentrationastratificationsaareaartificiallyaintroducedaintoaanainitiallyanonreactiveaflow,awhichainevitablyaneglectsatheaeffectaofaearly-stageacombustionaonasubsequentaauto-ignitionainitiationaandadevelopmentaduringaengine-knockingacombustion.aTherefore,aaamoreacomprehensiveastudyaisaneededatoabetteraunderstandathea complexaphenomenaaduringatheaautoignitionaprocess.aInasummary,aautoignitionamodeahasabeenaextensivelyastudiedainarecentaliteratureadueatoaitsasignificantaroleainaknockingacombustion.aDifferenta autoignitionamodesaexistadueatoatheanonuniformareactivityaofareactiveaflows.aTheaoperationalapeninsula,aconsistingaofaξaandaε,ahasabeenawidelyaadoptedatoaclassifyadifferentacombustionaregimesawithacleararegimeaboundariesainatermsaofaautoignitionamode.aMorearecentaresearchahasashownathatathermalaandaconcentrationastratifications,aasawellaasatheiracrosscoupling,acanasignificantlyaaffectatheadevelopmentaofaautoignitionamode.aHowever,apreviousastudiesahaveaneglectedatheaeffectaofaearlystageacombustionaonasubsequentaautoignitionainitiationaandadevelopmentaduringaengineknockingacombustion.

Theaautoignitionamodeaisadependentaonatheathermochemicalastateaofatheafuel/airamixture,aincludingafactorsasuchaasatemperature,apressure,aequivalencearatio,aandareactionaprogressaatatheamomentaofaautoignition.aDuringatheaearlyastagesaofaengineacombustion,atheathermochemicalastatusaofatheamixtureaisasignificantlyaaffectedabyavariousaphysicalaprocessesasuchaasaturbulenceaflow,apressureawaveadisturbance,aandaflameafrontacompressionaandaradiationa[12atoa15].aSeveralastudiesahaveashownathatapressureawaveapropagationacanainduceaaasignificantaincreaseainafreshagasesatemperature,aleadingatoaaadecreaseainaignitionadelayatime,aultimatelyaleadingatoadeflagrationatoadetonationa[9,15].aTheareflectionandaintersectionaofapressureawavesainaengineacylindersahaveabeenafoundatoabeacrucialaforalocalaautoignitionainitiationaandaknockingaformationa[15,19].aMoreover,amanyaopticalaexperimentsainaenginesaandarapidacompressionamachinesasuggestathatapressureawaveapropagationaandareflectionaunderaenclosedaconditionsaareacloselyaassociatedawithalocalaexplosiveaautoignitionanearwallaregionsa[17,18].aPreviousastudiesahaveashownathatatheaeffectaofapressureawaveadisturbanceaonaautoignitionalocationsavariesaunderadifferentathermodynamicastatesaandachamberalengthsa[19,20].aHowever,atheacriticalaroleaofapressureawaveastrengthainamodifyingaautoignitionamodeaandaknockingaintensityahasanotabeenathoroughlyastudied.aFurthermore,athereaisaaalackaofaliteraturearegardingatheaeffectsaofaflowadisturbanceaonaauto-ignitionamodeaandaknockingaintensityaunderaengine-likeaconditions.

Inasummary,atheaearlyastagesaofaengineacombustionasignificantlyaaffectatheathermochemicalastatusaofatheamixture,aandaphysicalaprocessesasuchaasaturbulenceaflowaandapressureawaveadisturbanceaplayaaacrucialaroleainamodifyingaautoignitionamodeaandaknockingaintensity.aFurtheraresearchaisaneededatofullyaunderstandatheaeffectsaofapressureawaveastrengthaandaflowadisturbanceaonaautoignitionamodeaandaknockingaintensityaunderaengine-likeaconditions.

Chapter 5

Methodology and Numerical Models

5.1 GoverningaEquations

Theaflowafieldsaare modelled withathe compressibleaNavier– Stokes equations and the equations of state of a thermally perfect gas. These are as follows:

where ρ is theadensity, u is the velocity vector, pais the pressure, δ is the unit tensor, τ is the viscous stress tensor, E is theatotal energy (E=ρⅇ+12ρ u.u), q is the heat flux vector, e is the internalaenergy, Ys is the mass fraction, Ds is the diffusionacoefficient, and ώ is the production rate of eachaspecies s. R is the universal gasaconstant (R = 8.314 J mol-1 K-1), T is the temperature, and Ms is the molar mass of each species. Here the subscript s = 1 ~ N, whereaN is the total number of species.

The viscous stress tensor s is represented by:

Whereaμaisatheaviscosityaofatheamixtures, aandaS is theasymmetric strainarateatensor. aTheaheat fluxavector q is modelled as follows:

Where κaisatheathermal conductivityaofatheamixturesaand hs is the enthalpyaof eachaspecies.

5.2 Numerical Model

The governing equations are compressible Navier-Stokes equations with mass conservation of chemical species and energy equations. To perform 2D DNS with high efficiency, PeleC [21], a compressible reactive flow solver with the AMReX library that can handle dynamic adaptive mesh refinement (AMR), was employed. Harten, Lax, van Leer, and Contact (HLLC) [22] was used as an approximate Riemann problem solver. The piecewise parabolic method (PPM) was employed as an interpolation method to reconstruct the values at the face [23]. The second-order central difference was used for the transport terms. The spectral deferred correction approach (SDC) [24,25], which is an iterative scheme, was employed as a time advance scheme, and the number of iterations was set as two. To achieve the 2D DNS for knocking experiments by further reducing computational cost, an efficientachemical equation solver, MACKS (Minimum ErroraAdaptation of ChemicalaKinetic Solver) [26], was developed and incorporated into PeleC.

Regarding chemical kinetics, the latest validatedareduced chemical kinetics [27–29] developed in the SIP EngineaProject in Japan [30] was applied. Because the fuel in this study isan-heptane, reactions related to iso-octane were excluded from the original kinetics. The numbers of chemicalaspecies and elementaryareactions after the above exclusion is 53 and 199, respectively. The validationacases of either full or reduced SIP kinetics [29] can beafound in the literature [27,28,31–33]. Additionally, since the chemical kinetics developed in the SIP needs to handle the PLOG function, Zero-RK [34] was incorporated into PeleC.

5.3 Computational Model

A direct numerical simulation (DNS) is a simulationain computational fluid dynamics (CFD) in which the compressibleaNavier–Stokes equations are numerically solved without anyaturbulence model. This means that the wholearange of spatial and temporal scalesaof the turbulence must be resolved. All the spatial scales of the turbulence must be resolvedain the computational mesh, from the smallestadissipative scales (Kolmogorov microscales), up to the integralascale L associated with the motions containing most of theakinetic energy. The Kolmogorov scale, η is given by

η=ν3ε14

Where ε  is the rate of kinetic energy dissipation and ν is the kinematic viscosity. On the other hand, the integral scaleausually depends on the spatial scaleaof the boundary conditions.

To satisfy these resolutionarequirements, the number of points N along a givenamesh direction with increments h must be:

Nh>L

so that the integralascale is contained within the computationaladomain and also

h≤η

so that the Kolmogorovascale can be resolved.

Since,

ε≈u'3L

where u ′ u' is the root mean square (RMS) aof the velocity, the previous relationsaimply that a three-dimensional DNS requires a number of mesh points N 3 N3 satisfying

N3≥Re2.25

Where Re R e  Re is the turbulent Reynolds number:

Re=u'Lν

Hence,atheamemoryastoragearequirementainaaaDNSagrowsaveryafastawithatheaReynoldsanumber.aInaaddition,agivenatheaextensiveamemoryanecessary,atheaintegrationaofatheasolutionainatimeamustabeadoneabyaanaexplicitamethod.aThisameansathatainaorderatoabeaaccurate,atheaintegration,aforamostadiscretizationamethods,amustabeadoneawithaaatimeastep,aΔt,asmallaenoughasuchathataaafluidaparticleamovesaonlyaaafractionaofatheameshaspacingahainaeachastep.aThatais,

C=u'Δth<1

(C  is here the Courant number). The total time intervalasimulated is generally proportional to the turbulenceatime scale τ given by,

τ=Lu'

Combining these relations, and the fact thatah h must be of the order of ηη, the numberaof time-integration steps must beaproportional to L / ( C η )L(Cη). On the other hand, from the definitions foraRe, η and L given above, it follows that:

Lη~Re0.75

and consequently, the numberaof time steps grows also as a poweralaw of the Reynolds number.

It can be seen that the number of floating-point operations required to complete the simulation is proportional to not only the number of mesh points but also the number of time steps. Thus, the number of operations grows as Re3 power lawR e 3.

Therefore,atheacomputationalacostaofaDNSaisaveryahigh,aevenaatalowaReynoldsanumbers.aForatheaReynoldsanumbersaencounteredainamostaindustrialaapplications,atheacomputationalapowerarequiredabyaaaDNSawouldaexceedatheacapacityaofatheamostasuperacomputersacurrentlyaavailable.aHowever,aDNSaformsaaaveryaimportantatoolainafundamentalaresearchainaturbulence.aUsingaDirectaNumericalaSimulation,aitaisapossibleatoaperforma“numericalaexperiments”aandaextractafromathemainformationawhichaisadifficultaoraimpossibleatoaobtainainathealaboratory,aallowingaaabetteraunderstandingaofatheaphysicsaofaturbulence.aAlso,aDNSaareausefulainatheadevelopmentaofaturbulenceamodelsaforapracticalaapplications,asuchaasasubgridascaleamodelsaforalargeaeddyasimulationa(LES)aandamodelsaforamethodsathatasolveatheaReynoldsaveragedaNavierStokesaequationsa(RANS).aThisaisadoneabyameansaofa“aapriori”aatests,ainawhichatheainputadataaforatheamodelaisatakenafromaaaDirectaNumericalasimulation,aorabya“aaposteriori”atests,ainawhichathearesultsaproducedabyatheamodelaareacomparedawithathoseaobtainedabyaDirectaNumericalaSimulations.a

5.4 Boundary Conditions

Aaconstantavolumeachambera(14a×a14a×a80amm)ausingaaastoichiometricanC7H16/O2/AramixtureawhereaO2aandaAramoleafractionaratioaofa21:79aataanainitialatemperatureaofa1150aKaandapressureaofa 0.304aMPaawasaused.aThealeft,abottom,arightaandaupperaboundaryaconditionsawereasetaasaadiabaticanonslipawalls.aForaignition,aaahotakernelaofa1400aKawithaaaradiusaofa1.0a×a10−3amawasasetaata xa=a2.5a×a10−3amaandaya=7.0a×a10-3am. Theatemperatureaandaradiusaofatheahotakernelaareatheasameaasathoseainatheapreviousastudies,awhichainvestigateatheaknockingaphenomenaaofanheptane.aTheadomainaofatheatwodimensionalasimulationa wasadefinedaasathearegionashownainaFig.a5.1.

A 2560×448 equally spaced orthogonal mesh was placed in the domain (80×10-3×14×10-3 m), and the mesh size is about 31.25×10-6 m. There, fine grids were placed on the flame surface by setting the AMR level as 3, and the minimum mesh size is about 3.906×10-6 m.

The simulation was also performed for the initial temperature-pressure combination of 1150 K-3atm and the initial temperature of 1000 K, keeping all other conditions the same in order to study the different forms of auto-ignition.

Figure 5.1: Schematic of the Computational Domain

5.5 Grid Convergence Study

Grid convergence study has been done in an earlier study using same numerical method but at a lower initial temperature of 480 K. The flame thickness in our case will be slightly smaller but the temperature gradients will also be smaller. Consequently the grid requirements will not be very different from the one at lower temperatures. The time step is governed by CFL number. The earlier work is described here to show the grid convergence study. AaresearchagroupaataKyushuaUniversity,aJapan,aconductedaanaexperimentatargetingatheaknockingaphenomenon.aTheyautilizedaaaconstantavolumeachambera(14a×a14a×a80amm)ausingaaastoichiometrican-C7H16/O2/AramixtureawhereaO2aandaAramoleafractionaratioaofa21:79aataanainitialatemperatureaofa480aKaandapressureaofa0.33aMPaa[35].aTheadomainaofatheatwodimensionalasimulationawasadefinedaasathearegionashownainaFig.a5.1.aTheseainitialaconditionsawereabasedaonatheapreviousaexperimentsainawhichatheastrongaknockaonsetawasaobservedaonlyaataanainitialatemperatureaofa480aKa[36].aInacontrastatoapreviousaexperimentsawhichausedaonepointasparkaignitionaataaasingleaendaofatheachamber,atheatargetaexperimentaemployedatwopointasparkaignitionaatabothaendsaofatheachamberatoainvestigateatheaeffectaofasparkaignitionaonaknockaonset.aDueatoatheasymmetricalanatureaofatheachamber,atheaone-dimensionalasimulationacomputationaladomainawasadefinedaasathearedalineainaFig.a5.2,awhileatheato-dimensionalasimulationacomputationaladomainawasadefinedaasathearegionasurroundedabyablueadottedalinesainaFig.a5.2.aToainitiateaignition,aaahotakernelawithaaatemperatureaofa1400aKaandaaaradiusaofa1.0a×a10−3amawasaplacedaataxa=a2.5a×a10−3amaandaya=a7.0a×a10-3am. Theatemperatureaandaradiusaofatheahotakernelaareaconsistentawithapreviousastudiesa[37,38],athataexaminedatheaknockingaphenomenaaofan-heptane. Thealeftaboundaryaconditionawasasetaasatheaadiabaticanonslipawallaandathearightaboundaryawasasetaasasymmetricainatheaonedimensionalasimulation.aInatheatwodimensionalasimulation,athealeftaandabottomaboundaryaconditionsawereasetaasaadiabaticanonslipawallsawhileathearightaandaupperaboundaryawallsaawereasetaasasymmetric.aP1aandaP2ainaFig.a5.2aareatheapressureaprobeapoints.aTheapressureasensorainatheaexperimentawasaequippedaataP2.

Figure 5.2: Schematic of theaOne- and Two-DimensionalaComputational Domains

Toadetermineatheaappropriateameshasize,aaagridaconvergenceastudyawasaconductedausingaonedimensionalasimulationsabeforeacomparingawithatheaexperiments.aTheameshanumberawithoutaAdaptiveaMeshRefinementa(AMR)awasasetatoa5120aina4a×a10−2am,aandatheaAMRalevelsawereasetatoa0,a1aanda2,a i.e.,atheaminimumameshasizesaΔxminawerea7.8125a×a10−6,a3.90625a×a10−6,aanda1.953125a×a10−6am,arespectively.aTheapressureahistoryaataP1aisashownainaFig.a5.3.aNoteathatasinceatheatimeastepsawereanotaconstantainatheapresentacomputation,aallatheapressureahistoriesainathisastudyawerearesampledatoa1aMHz,awhichaisatheasamplingafrequencyausedainatheaexperimenta[35].aBasedaonatheacomparisonaofatheapeakapressureaandaknockaonsetavalues,aaasufficientaaccuracyawasaobtainedawithaΔxmina=a3.90625a×a10−6am.aAsaaaresult,athisameshasizeawasausedainasubsequentasimulations,aincludingatheatwo-dimensionalasimulation,awhichahadaaameshanumberawithoutaAMRasetatoa5120a×a896aandaanaAMRalevelasetatoa1.aTheatimeastepsainatheaoneaandatwodimensionalasimulationsawerealimitedabyaaaCourantanumberaofa0.25,aandatheyawereaarounda2a×a10−9asaforaΔxmina=a3.90625a×a10−6am.aTheavaluesa ofaΔxminainatheafigureaareashownawithaoneasignificantadigit.

Figure 5.3: Pressure History at P1 using Δxmin = 1.953125 × 10−6 - 7.8125 × 10−6 m

Thisasectionabeginsabyapresentingaandadiscussingatheapressureahistoriesainatheaoneaandatwodimensionalasimulations.aTheapressureaandadP/dtahistoriesaatatheapressureaprobeapointsaP1aandaP2aareaillustratedaInaFig.a5.3aandaFig.a5.4awherea(a)arepresentsatheapressureahistoriesaforaoneaandatwodimensionalasimulationsaasawellaasatheaexperimentaanda(b)ashowsatheadP/dtahistoriesaforatheaoneaandatwodimensionalasimulations.aNoteathatadP/dtawasacalculatedawithatheapressureahistoriesaresampledaata1aMHzaandalowpassafilteredaata1akHzatoaindicateatheaimportantaeventsainatheacomputations.aTheapresentationaofadP/dtaisarelevantahereabecauseaitacanaprovideainsightsaintoatheatimingsaofacriticalaeventsasuchaasaflameacontactawithawalls,acoolaflameaignitionabeforeatheaknockaonset,aandatheaknockaonsetaitself.aThearesultsaofatheatwodimensionalasimulationaataP1aandaP2aareashownainaFig.a5.4aimplyingathatatheadifferenceainatheapressureaprobeapointsawasanegligible.aThisameansathatapressureahistoriesaataP1aandaP2aalmostamatchaeachaother.aThisaimpliesathataeitheraofaP1aoraP2acanabeausedatoamakeatheacomparison.aTherefore,ainatheafollowing,atheapressureahistoryaataP1ainatheatwodimensionalasimulationawillabeausedaforatheacomparisonawithatheaone-dimensionalaresults.

Figure 5.4: Pressure Histories at P1 and P2

Theapressureahistoryainatheaonedimensionalasimulationsabeforeatheaknockaonsetadisplayedamoreapronouncedapressureaoscillationsainatheaflameapropagationaphaseathanatheaexperiment,aasaevidentafromatheagreenacurveainaFig.a5.4.aTheseaoscillationsawereasimilaratoathoseaseenainatheasimulationsabyaYuaetaal.a[20],aindicatingathatatheyawereacausedabyatheapropagationaofapressureawavesainatheaclosedachamber.aWhileaapplyingaaa1akHzalowpassafilteracouldapotentiallyaeliminateatheseaoscillationsasinceatheirafrequencyawasamuchahigherathana1akHz,aaaonedimensionalasimulationainaFig.a5.5aconfirmedatheaexistenceaofatheastrongapressureaoscillation.aTherefore,aitaappearsathatainatheaonedimensionalasimulations,atheashockawaveaproducedabyatheainitialaignitionawillamostalikelyacontinueatoapropagateaoveraanaextendedaperiodawithoutasignificantadecay,awhichadiffersafromatheaexperimentalaobservations.a Furthermore,atheacomputedatimingaofatheaknockaonsetabyatheaonedimensionalasimulationawasa7.53ams,awhichaisasignificantlyalongerathanatheaexperimentalaknockaonsetatimeaofa5.84ams.aThisadisparityamayabeaattributedatoatheainabilityaofatheaonedimensionalasimulationatoatakeaintoaaccountatheatypicalatransitionahistoryainapremixedaflameapropagation,asuchaasasphericalaflame,afingeraflame,aplanaraflame,atulipaflame,aandawrinkledaflame,aasacanabeadoneainatheatwodimensionalasimulation.aConsequently,atheaerroraaccumulatedauntilatheaknockaonsetawasasmallerainatheatwodimensionalasimulationathanatheaone-dimensionalasimulationaandacloseratoatheaexperimentalavalue.

Figure 5.5: dP/dt Histories at P1

Theatwodimensionalasimulationaperformedainatheapresentastudyaaccuratelyareproducedatheapressureahistoriesaobservedaduringaknockingaexperimentsaandadidanotaexhibitatheastrongapressureaoscillationsaobservedainaonedimensionalasimulations.aTheacomputedatimingaofatheapressureaincreaseadueatoaflameacontactawithatheabottomawallawasa1.76amsainaFig.a5.5,awhichawasaconsistentawithatheafaintapressureaincreaseaobservedainatheashadedaareaaonatheaexperimentalapressureahistoryainaFig.a5.4.aTheatimingaofatheaflameacontactawithatheawallsawasaestimatedatoabeaapproximatelya2.4amsainatheaexperiment,asuggestingathatatheasimulationaresultsawereahighlyaaccurate.aTheacoolaflameaignitionatimingawasaestimatedatoabea5.04amsainatheaexperimentaanda5.30amsainatheasimulation,aindicatingaaagoodaagreementabetweenatheatwo.aAdditionally,atheacomputedatimingaofatheaknockaonsetawasa5.76ams,awhichawasaslightlyafasterathanatheaexperimentalavalueaofa5.83ams,awithaanaerroraofalessathana2%,aindicatingagoodaagreementabetweenatheasimulationaandatheaexperiment.aY.aMorii,aA.K.aDubey,aH.aNakamura,aandaK.aMarutaa[39]aconductedaaastudyatoaexploreatheacharacteristicafeaturesaofatheaknockingaprocess,autilizingatheapressureahistoryaandatheadensityagradientainatheatwodimensionalasimulationaandacomparingaitawithaexperimentalaSchlierenaimages.aThey discovered that the density gradient revealed a bright flame in the early stages of knocking, which gradually weakened as the knock progressed. This finding is in agreement with the pressure history, which showed a rapid increase in pressure followed by a gradual decrease as the knock progressed. The study by Morii et al. [39] offers valuable insights into the characteristics of knocking processes, providing a deeper understanding of the underlying mechanisms. The agreement between the simulation and experimental results demonstrated in the present study, along with the findings of Morii et al. [39], demonstrates the importance of utilizing advanced simulation techniques to investigate complex phenomena such as knocking in internal combustion engines.

Chapter 6

Results and Discussions

6.1 Auto-Ignition Modes

Three forms of local auto-ignition have been identified through previous research. Various initial conditions heavily influence the mode of formation of the Auto-Ignition zone and its location. The initial temperatureaof the reactant mixture, the initial pressure inside the chamber and the initial temperature of the hot ignition kernel are just some of the factors that play the most significant role in the location of the Auto-Ignition zone. It was seen that at T0=1150 K and P = 3 atm, there were two Auto-Ignition zones formed. One just ahead of the SI flame front and one near the end wall. Whereas, atainitialaconditionsaof T0=1150 KaandaP = 10 atm, only one Auto-Ignition zone was seen to be formed at a location just ahead of the SI flameafront. Other conditions included T0=1000 K, where only one Auto-Ignition zone was seen to be formed near the end wall. Therefore, as the initial temperatureaincreases, there is aatransition in the auto-ignitionalocation fromatheanear-wallaregionato thealocation ahead of theaSIaflameafront. These conditions were the central focus of the current research.

T0=1150 K P=10atm

T0=1150 K P=3atm

T0=1000 K

Figure 6.1: Scenarios of LocalaAuto-Ignition and FlameaPropagation at DifferentaInitial Temperatures

In order to study these results, two different post-processing software were used to understand the results. Paraview was used to generate the data points of temperature, pressure, density, and mass fraction of various chemical compounds in the burnt mixture concerning the X-axis and Y-axis, keeping the other constant at different timesteps. With the help of MS Excel, all the data points were plotted on the graphs to analyse the behaviour of the parameters and their peak values. An extensive script was developed using Python language to automate the contours generation using VisIT, another CFD post-processing software (Appendix-B).

6.2 Autoignition Modes at 3 atm and 1150 K

Temperature distribution in the domain is presented here to visualise these modes. Figure 6.1 shows the temperature distributions for the case of T0=1150K at various instants of flame propagation.

Figure 6.2: Temperature Contours at Different Timesteps.

We can easily see from the figure that the flame front, due to the initial ignitionaof the mixture, propagates as a circular flame until the flame touches the left wall. After that flame propagates as a finger flame. These are the usual events when flame travels in closed constant volume chamber. Soon after, Auto-Ignition is observed in theaend-gasamixture. Itais observed thatatheaAuto-Ignition starts at around t = 401µs. There are two Auto-Ignition zones, one in front of the flame and 2nd near the end wall. It may not be the case in all the setups; the change in initial temperature will change the location and number of Auto-Ignition zones. We can also confirm the same by analysing the Pressure contours at different timesteps. The following contours are shown in Figure 6.2.

Figure 6.3: Pressure Contours at Different Timesteps.

Here also, it can be observed that at a timestep of approximately 401µs, the Auto-Ignition zones have already started. To pinpoint the accurate time at which the Auto-Ignition zones begin, we analysed the temperature and pressure plots about the centerline ofatheadomain. Figurea6.3ashowsathe plots of temperature and pressure atadifferent timesteps about the centerline, i.e., Y = 0.7. The sudden drop in temperature graph represents the location of the flame front. The second local maxima in the graph is due to the auto-ignition inside the domain. A similar peak in pressure also indicates the Auto-Ignition zone’s locations. The travel of these peaks as time increases signifies that the Auto-Ignition flame is expanding and burning the fuel before the ignited flame can reach the particular location.

Figure 6.4: Temperature and Pressure Plots about Centreline at Different Timesteps @ 3atm 1150K

As observed from the above discussion, we can now narrow our time domain from 390.29 μs to 403.3 μs. We then plotted the temperature and pressure contours and graphs for this time range and tried to understand the behaviour of the peak to determine the exact location of the Auto-Ignition zones.

Figure 6.5 shows the temperature distribution at significant instants for an initialatemperature of 1150 K. Here, two Auto-Ignition centres are observed at t=401.123 μs, one ahead of the flame front and one in the end-gas. Auto-Ignition occurs before the flame touches the side walls. The auto-ignitionaahead of the flameafront is stronger than the one in the end gas.

Figure 6.5: Temperature Distribution at Various Instants Clarifying the Auto-Ignition Mode

The Temperature and Pressure of these time steps will help us determine the precise location of the Auto-Ignition Zones. Figure 6.6 shows the temperature and pressure v/s X-Axis plots about the centerline. From these plots, it can be concluded that the Auto-Ignition started at about 393µs and the location at that time is about X = 2.594 and X = 7.985. We then plot the temperature and pressure graphs with respect to the Y-axis. We plotted the graphs for five different timesteps, which include t = 399.8 µs, t = 400.55 µs, t=401.12 µs, t = 401.68 µs, and t = 402.23 µs.

Figure 6.6: Plots of Temperature and Pressure at Important Timesteps.

Figure 6.7 shows the graphical representations of the Temperature and Pressure variation with respect to the Y-axis at X = 2.594. A peak started emerging at time t = 400.55 µs and became strong at time t = 401.12 µs. It is also seen that the peak is travelling with respect to Y-axis. The two Auto-Ignition zones are visible in the following graphs. This is because of the symmetric domain about the centerline. We can conclude from the curves that the Auto-Ignition zones in Y-axis are near Y = 0.2128 and Y = 1.1872.

Figure 6.7: Temperature Plots (left) and Pressure Plots (right) at Certain Timesteps

We plotted the temperature and pressure change rate to understand the auto-ignition zone and pinpoint the Y location of the zone. It also helps us calculate the flame speed of the auto-ignition zone and along the Y-axis. Figure 6.8 shows dP/dt and dT/dt graphs in a similar timestep range.

Figure 6.8: Rate of Change of Pressure(dP/dt) and Temperature(dT/dt) w.r.t. Y-axis

Another auto-ignition zone observed from the previous contours and plots was near the wall, i.e., at X =7.985. A similar analysis was performed about the X value as it was in the case of X=2.594. Here also, the graphs about Y-axis were plotted and analysed for auto-ignition zones. The same value range of time is considered. The graphs can be seen in Figure 6.9, left (Temperature plots) and right (Pressure plots). The peak in temperature has started to arise at t = 401.68µs. A small valued peak is also significant in the case of auto-ignition. Also, a similar case can be verified using a pressure curve. This concludes that the auto-ignition zones are at Y = 0.2128 and Y = 1.1872. The values are similar to the previous case.

Figure 6.9: Temperature Plots (left) and Pressure Plots (right) @ X = 7.985

We plotted the rate of change of temperature and pressure to understand the auto-ignition zone and pinpoint the Y location of the zone for this case also. It also helps us calculate the flame speed of the auto-ignition zone and along the Y-axis. Figure 6.10 shows dP/dt and dT/dt graphs in a similar timestep range.

Figure 6.10: Rate of Change of Pressure(dP/dt) and Temperature(dT/dt) w.r.t. Y-axis @ X = 7.985

6.2.1 Numerical Estimation of Laminar Flame Speed

A numerical approach was first used to estimate the laminar flame sleep of the SI flame front using the Canteraasoftware package. Cantera is an open-source collection of tools foraproblems involving chemical kinetics, athermodynamics, andatransport processes for solving chemicallyareacting laminar flows. Cantera automates theachemical kinetic, thermodynamic, aand transport calculations so that the users can efficiently incorporate detailedachemical thermo-kinetics and transportamodels into their calculations. For the stoichiometric n-heptane mixture,  the “Gasoline PRF 2.0 Reduced Mechanism” was used. The mechanism file consists of lists of all the species and their interfaces, a list of all the elements, and a list of all the reactions with corresponding reaction rates. Constituents of the same can be seen in Appendix C.

The laminar flame speed was found to be 29.83 m/s.

6.2.2 Numerical Estimation of C-J Detonation Speed

TheaChapmanJouguetaconditionaholdsaapproximatelyainadetonationawavesainahighaexplosives.aIta statesathatatheadetonationapropagatesaataaavelocityaatawhichatheareactingagasesajustareachasonica velocitya(inatheaframeaofathealeadingashockawave)aasatheareactionaceases.

CanteraawasausedaalongawithatheaShockaandaDetonationaToolboxa(SDaToolbox).aTheaShocka&aDetonationaToolboxaisaanaopensourceasoftwarealibraryathataenablesatheasolutionaofastandardaproblemsa foragas-phaseaexplosionsausingarealisticathermochemistryaandadetailedachemicalakinetics.

Detonationawavesaareamultidimensional,aaunsteadyacombustionawavesathatapropagateaataaasupersonicavelocity.aTheyaareacharacterisedabyatheastrongacouplingabetweenathealeadingashockafrontaandathea chemicalareactionazonebehindait.aTheaautoignitionaofatheareactiveamixtureaisaensuredabyaadiabaticashockacompression.aAsaaaresult,atheaexpansionainducedabyatheacombustionaprocessasustainsashockapropagation.aTheasimplestamodelaconsidersaaadetonationawaveaconsistingaofaaasteady,aplanarashockawaveafollowedabyaaareactionazone.aAtathermodynamicaequilibrium,atheaflowabecomesasonicawitharespectatoathealeadingashockawave.aThisamodelawasaindependentlyaproposedabyaZel’dovich,avonaNeumann,aandaDoeringaandaisareferredatoaasatheaZNDamodel.aWithatheaincreaseaofacomputationalaresources,anumericalaimplementationaofatheaZNDamodelainaChemkinabecameapossible.aTheaZNDamodelawasalateraimplementedainaCanteraaandaisacurrentlyaavailableainatheaShockaandaDetonationaToolboxa(SDaToolbox).

The steady detonation model was used, the formulation of which is:

Where,

Withat:atime,aω:aflowaspeedainatheadetonationaframe,aρ:adensity,aP:apressure,aσ:athermicity,aM:a Machanumber,aη=1a−aM2:asonicaparameter,aYi:amassafraction,aωi:asourceaterm,aN:anumberaofa species,aW:ameanamolaramass,aWi:amolaramass,ahi:aenthalpy,aCP:aheatacapacityaataconstantapressure,aandaT:atemperature.a

First,atheaZNDamodelaisasolvedausingatheaSDToolbox.aTheacodeacalculatesatheaChapman-Jougueta(CJ)adetonationavelocityadependingaonatheainitialatemperatureaandapressureainputs.aUsinga thisavelocity,atheapostshock/vonaNeumannastateaisaobtainedaandaisausedaasatheainitialastateafora integratingaEquationa1.aItaisanotedathataaanewaCJavelocityasolver,abasedaonathearootasolveraofa SciPy,awasadesignedatoaimproveaefficiency.aSecondly,asinceatheasolutionaofatheaZNDamodelaprovidesatheastateaofatheamixtureaataeveryainstant,atheaHRR,aRoP,aandasensitivityacoefficientacanabea calculatedaaccordingly.aForaHRRaandaRoP,a∆rH◦,arjaandaνiacanabeadirectlyaobtainedafromaCantera.a

Foratheasensitivityacoefficient,athearateaconstantaofatheatargetedareactionaisaperturbedabya1%.aThea perturbedareactionamodelaisathenausedatoasolveatheaZNDamodel.aTheasensitivityacoefficientaisa finallyaapproximatedawithaaaforwardadifference.

Again, the “Gasoline PRF 2.0 Reduced Mechanism” was used. The C-J Detonation speed was found to be 1342 m/s.

6.2.3 Analytical Estimation of Laminar Flame Speed

AapioneeringaworkabyaZel’dovicha [5] ashowedathat, athereaareadifferentaauto-ignitionamodesa (i.e., subsonicadeflagration, adetonation, aandasupersonicadeflagration) acausedaby the non-uniform reactivity ofareactiveaflows. aInathisaclassification, athermalaexplosionarepresentsaaalimitingacaseaofasupersonic auto-ignitionawithaanainfiniteareactionafrontaspeed.

Studying the OH mass fraction profile helps determine the flame fronts’ propagation. Peaks in the curve determine the presence of a flame front, and a steep rise, when observed, implies ongoing reactions in that area. Fig. 6.11 and Fig. 6.12 shows the temperature andaOH mass fraction profile along the centerline at the auto-ignition time. It is observedathat temperature and OH mass fraction shows a global maximum at the location of the flame front and a local maximum at the location of the auto-ignition centre. It is observed that auto-ignition occurs both at the end-wall region and also at the location just ahead of the SI flame front.

Toafurtheravalidateatheaaboveaconclusion,aFig.a6.13agivesathearesultsaofaAIareactionafrontaspeed. aHereaAIareactionafrontaisaidentifiedathroughatheapresenceaofalocalamaximaainatheaOHamassafractionacurveainathealocal mixture. aTheacorrespondingapropagationaspeedaofatheaAIareactionafront is definedaasathearateaofachangeaofalocationaofathisamaximaaperaunitatime. aItaisaobserved that after end-gasaauto-ignition, atheapropagationaspeedaofatheaAIareactionafrontaincreasesafrom theasubsonic to theasupersonicalevel. aTherefore, athereaisaaapossibilityathatatheaAIareactionafrontaandapressureawave interactawithaeachaother. aSpecifically, atheaAIareactionafrontaincreasesafromatheasubsonicalevelato approximatelyav = 1500 m/s, aclosingatoatheaC-Jadetonation speedaat the corresponding thermodynamic state. aTheseaobservationsaagainademonstrateaaamodeatransitionaforaend-gasaauto-ignition with theadecreaseainastrengthaofapressureawaveadisturbance.

Figure 6.11: Temperature Variation near the Flame Front (Zooming the peak as we move down)

Figure 6.12: OH Mass Fraction  Variation near the Flame Front (Zooming the peak as we move down)

When the temperatureaandaOHamassafraction curves are plotted alongside on the same graph for some specific time steps, it can be observed that there is a one-time step difference (Δt=2μs) between their global peaks. This can be attributed to a time lag between the reaction starting (OH mass fraction attaining its peak value) and a temperature rise at that point in the chamber (Temperature curve attaining its peak value), as seen in Fig. 6.14.

Figure 6.14: Comparison of AI Zones based on Temperature and Mass fraction of OH

Dueatoatheainherentlyadifferentaphysics,atheavelocityascalesaassociatedawithatheaflameafrontapropagation,apressureawaveaandaautoignitionaofathealocalamixtureaarealargelyadifferent.aItaisaobservedathatabeforeaendgasaautoignition,atheapressureawaveapropagatesabackwardsaandaforwardainsideatheachamberawithaaalocalasoundaspeedaofaarounda900am/s.aConsequently,atheaSIaflameafrontapropagationaprofileaexhibitsaanaoscillatingabehaviourabutawithaaameanadeflagrationaflameaspeedaofa32am/s.aOnathea contrary,atheaAutoIgnitionareactionafrontapropagatesamuchafaster.aOnceaautoignitionaoccurs,atheacorrespondingareactionafrontacouldapropagateaataspeedaupatoaovera1200am/s,astillalowerathanaaafullyadevelopedasteadyaCJadetonationaspeedaofaabouta1300am/saforatheasameamixtureaundera3aatmaanda1150aK.aItaisaobservedathatabeforeaautoignition,athealocalamixtureahasabeenadisturbedabyaaapressureawaveabya5atoa10atimes,aandaforaeachatime,athealocalatemperatureaandapressureawillabeaincreasedabyatheareflectionaofatheapressureawaveafromatheawall.aDueatoatheasensitivityaofachemicalareactionsatoatemperatureaandapressure,atheseafluctuationsamakeaignitionamuchafasterathanaunderaconstantastateaconditions,ademonstratingatheasignificantaeffectaofapressureawavesaonatheaendgasaautoignitionaprocess.aOnatheaotherahand,atheaoccurrenceaofaautoignitionanearatheawallageneratesaaamuchahigheramagnitudeapressureawave,awhichafurtheraaffectsatheaSIaflameapropagation.

6.2.4 Quantification in Bradley’s Diagram

Toaquantifyatheaauto-ignitionamodeatransitionaandaknockingaintensityamodifiedabyapressureawave disturbance, aBradley’sadiagrama[6-8]aisaintroducedahereatoaevaluateatheacomputationacaseainathe currentawork. aInaBradley’sadiagram, thereaareatwoanon-dimensionalaparameters (ξ, ɛ), whereaξ describesatheacouplingabetweenaacousticawaveaandareactionafront, aandaɛaevaluatesathearapidityaof chemicalaenergyareleaseawithinaaahotaspot,

Whereaaaisasoundaspeed, uaiaisatheaspeedaofaauto-ignitionareactionafront, τaiaisaauto-ignitionadelay time, xaisatheaspatialaco-ordinateaofatheareactiveazone, ar0aisathearadiusaofahot-spot, aandaτeaisathe excitationatimeadefinedaasatheatimeaintervalabetweena5%aandamaximumaheatareleasearate. aItashould beanotedathata∂τai/∂x, the ignitionadelayatimeagradient, aallowsaforatheatransientatemperature, apressureaandaspeciesaconcentrationaatatheaauto-ignitionamomentainathisawork. aHereahot-spotaradius isaobtainedathroughaOHamassafractionadistribution, aandaknockingaintensityaisadefinedaas the pressure peakaofatheaAIareactionafront. aToaobtainatheaignitionadelayatimeaofathealocalamixture, abothathe transientathermodynamicastateaandaspeciesaconcentrationaofaallagridanodesain the computation domain is extractedaand thenautilisedainatheaCHEMKINaSENKINapackageaforacalculation.

6.3 Autoignition Modes at High-Pressure Conditions

To explain the pressure effect on auto-ignition mode, a simulation was done at an initial pressure of 10 atm. An initial temperature of 1150 K was chosen as this condition showed the most complex auto-ignition mode. Temperature distribution in the domain is presented here to visualise the modes. Fig. 6.16 shows the pressure distributions for the case of T0=1150K at various instants of flame propagation at a higher pressure of 10 atm.

Figure 6.16: Pressure Contours at Different Instances @ 10 atm 1150K

Fig. 6.17 shows the temperature contours for 1150K and 10 atm at different timesteps. The contours suggest that there exists only one auto-ignition zone inside the piston cylinder system. The auto-ignition zone is in front of the flame front at a timestep of around t = 228.27µs. It suggests that when the initial pressure is increased, the auto-ignition tends to start earlier than in lower-pressure conditions. It can also be concluded that the auto-ignition centre is at X = 2.594 in the case of 3atm, but in the case of 10 atm, it appears at X<2, which means that the auto-ignition centre is shifted towards the flame front.

Figure 6.17: Temperature Contours at Different Instances @ 10atm 1150K

Figure 6.18 shows the temperature and pressure plots at different time instances. Here, we can see that the sudden drop in the temperature shows the flame front, and the local maximum peak signifies the autoignition zone. The timestep at which this peak is captured is t = 228.89µs. So, we can say that the autoignition starts after 220µs of the flame ignited and at a location of X between 1 and 2. A similar analysis can be performed with Y plots also in the future to pinpoint the autoignition centre.

We can also analyse the pressure plots to understand the autoignition one. The peak in the graph signifies the location of the auto-ignition centre. Further, we can plot the graphs for timestep around 220µs and find the precise location and time of onset of autoignition.

Figure 6.18: Temperature and Pressure Plots about Centreline at Different Timesteps @ 10atm 1150K

Conclusion

Aatwo-dimensionalalaboratory-scaleaDNSaofaknockingaexperimentainaaastoichiometrican C7H16/O2/Aramixtureaatatheaenginelikeaconditionsaofaloweratemperatureaandapressureawithathealatest reducedakineticsawasarealisedausingaanaefficientainhouseasolver,aMACKS,acombinedawithaPeleC.a Theacomparisonaofatheapressureahistoriesafoundathatatheatwodimensionalasimulationawasaableatoa reproduceatheacharacteristicaevents,asuchaasatheacoolaflameaignitionaandatheaknockaonsetaobserveda inatheaexperiment.aTheasameacanabeaexploitedaforahighatemperatureaandapressureaconditionsafora whichaconductingaexperimentsaisamuchamoreadifficult.

Through this study, the behaviour of pressure waves due to the end gas auto-ignition flame front inside an n-heptane-fueled constant volume chamber under different conditions was understood. The initial pressure and temperature conditions are varied to understand the phenomenon under different conditions. At 1150 K 3atm, two distinct Auto-Ignition zones are observed: one slightly ahead of the SI flame front and one in the end gas. At 1150 K 10atm, the Auto-Ignition zone is observed only ahead of the flame front. First, pressure wave disturbanceais essential for combustion under enclosed conditions. Second,apressureawaveadisturbanceadoesanotaaffectatheaautoignitionalocationainareactiveaflowsabutasignificantlyaimpactsaautoignitionatimingaandamodeadevelopment.aAsatheaweaknessaofapressureawaveastrengthaataeachainitialatemperature,athreeaautoignitionamodesa(i.e.,adetonation,amixedamodeaandasupersonicadeflagration)ahaveabeenasequentiallyaobserved.aThisaisafurtheravalidatedabyatheapropagationaspeedaofatheaautoignitionareactionafront.aTheaessentialaroleaofapressureawaveadisturbanceainatheaautoignitionamodeatransitionacanabeaattributedatoatheapartialareactionaofathealocalamixtureaaheadaofatheaauto-ignitionareactionafront.a

After identifying the mode of auto-ignition, time for onset of auto-ignition was then estimated using contours of pressure and temperature. Plots for temperature, pressure and mass fraction f OH were plotted with time-steps around that time of auto-ignition. Peaks of the respective curves were studied in order to estimate the time duration of the auto-ignition, and these were also used to estimate the laminar flame speed of the SI and the AI flame fronts which was calculated as the distance travelled by the peaks per unit time. Contours were studied to study the AI flame propagation in that respective time frame. After the location of the AI zone was found w.r.t X axis, pots were generated and studied for the variation of the various parameters with Y axis. Peaks were again found from the plots, and these were used to estimate the exact location of the AI centres in the 2D domain. Plots were also generated for the rate of change of temperature and pressure in order to better understand the influence of these parameters on the AI mode and also the center of the AI. The laminar flame speed for both the SI and the AI flame fronts were calculated numerically using Cantera software package. CJ detonation speed for the initial conditions was also calculated using the Shock & Detonation Toolbox in order to study the exact auto-ignition mode.

Finally,aBradley’sadiagramaisaintroducedatoaquantifyaautoignitionamodeatransitionaandaknockingaintensity.aItashowsathatatheainitialathermodynamicastateaandaautoignitionamodeacanaaffectaknockingaintensity,abutatheamodificationsainaknockingaintensityabyapressureawaveadisturbanceaareamainlyaascribedtoaautoignitionamodeatransition.aWithaautoignitionamodeatransitionafromadetonationatoasupersonicadeflagration,aknockingaintensityavariesafromaaasufficientlyahighalevelasimilaratoasuperknockatoaconventionalaknock.aMeanwhile,aitaisafoundathatasomeadeviationsadoaexistadueatoainitiallynonreactiveaassumptionsaforaBradley’sadiagram;ahowever,aitaisaqualitativelyaconsistentawithatheadistributionaofacombustionaregimesainatheadiagramaforaauto-ignitionamodeaandaitsatransition.

References

[1]	Kimitoshi Tanoue, Taishu Jimoto, Takanori Kimura, Misato Yamamoto, Jun Hashimoto, Effect of initial temperature and fuel properties on knock characteristics in a rapid compression and expansion machine, Proceedings of the Combustion Institute Volume 36, Issue 3, 2017, Pages 3523-3531

[2]	J. Rudloff, J.-M. Zaccardi, S. Richard, J.M. Anderlohr, Analysis of pre-ignition in highly charged SI engines: Emphasis on the auto-ignition mode, Proceedings of the Combustion Institute Volume 34, Issue 2, 2013, Pages 2959-2967

[3]	Weiqi Sun, Sang Hee Won, Xiaolong Gou, Yiguang Ju, Multi-scale modelling of dynamics and ignition to flame transitions of high-pressure stratified n-heptane/toluene mixtures, Proceedings of the Combustion Institute Volume 35, Issue 1, 2015, Pages 1049-1056

[4]	Andrew B. Mansfield, Margaret S. Wooldridge, High-pressure low-temperature ignition behaviour of syngas mixtures, Combustion and Flame Volume 161, Issue 9, September 2014, Pages 2242-2251

[5]	Ya.B. Zeldovich, Regime classification of an exothermic reaction with non-uniform initial conditions, Combustion and Flame Volume 39, Issue 2, October 1980, Pages 211-214

[6]	XJ Gu, D.R. Emerson, D. Bradley, Modes of reaction front propagation from hot spots, Combustion and Flame Volume 133, Issues 1–2, April 2003, Pages 63-74

[7]	D. Bradley, G.T. Kalghatgi, Influence of auto-ignition delay time characteristics of different fuels on pressure waves and knock in reciprocating engines, Combustion and Flame Volume 156, Issue 12, December 2009, Pages 2307-2318

[8]	Luke Bates, Derek Bradley, Günter Paczko, Norbert Peters, Engine hot spots: Modes of auto-ignition and reaction propagation, Combustion and Flame Volume 166, April 2016, Pages 80-85

[9]	A. Robert, S. Richard, O. Colin, T. Poinsot, LES study of deflagration to detonation mechanisms in a downsized spark ignition engine, Combustion and Flame Volume 162, Issue 7, July 2015, Pages 2788-2807

[10]	Peng Dai, Zheng Chen, Shiyi Chen, Yiguang Ju, Numerical experiments on reaction front propagation in n-heptane/air mixture with temperature gradient, Proceedings of the Combustion Institute Volume 35, Issue 3, 2015, Pages 3045-3052

[11]	Tianhan Zhang, Weiqi Sun, Yiguang Ju, Multi-scale modelling of detonation formation with concentration and temperature gradients in n-heptane/air mixtures, Proceedings of the Combustion Institute Volume 36, Issue 1, 2017, Pages 1539-1547

[12]	Pinaki Pal, Mauro Valorani, Paul G. Arias, Im Hong G., Margaret S. Wooldridge, Pietro P. Ciottoli, Riccardo M. Galassi, Computational characterisation of ignition regimes in a syngas/air mixture with temperature fluctuations, Proceedings of the Combustion Institute Volume 36, Issue 3, 2017, Pages 3705-3716

[13]	Pan Jiaying, Wei Haiqiao, Shu Gequn, Chen Zheng, Zhao Peng, The role of low-temperature chemistry in combustion mode development under elevated pressures, Combustion and Flame Volume 174, December 2016, Pages 179-193

[14]	Mikhail F. Ivanov a, Alexey D. Kiverin a, Mikhail A. Liberman, Ignition of deflagration and detonation ahead of the flame due to radiative preheating of suspended micro particles, Combustion and Flame Volume 162, Issue 10, October 2015, Pages 3612-3621

[15]	M. A. Liberman ,M. F. Ivanov,O. E. Peil, D. M. Valiev &L. -E. Eriksson, NUMERICAL MODELING OF THE PROPAGATING FLAME AND KNOCK OCCURRENCE IN SPARK-IGNITION ENGINES, Combustion Science and Technology Volume 177, 2004 - Issue 1, Pages 151-182

[16]	M. A. Liberman ,M. F. Ivanov,O. E. Peil, D. M. Valiev &L. -E. Eriksson, HOT SPOT FORMATION BY THE PROPAGATING FLAME AND THE INFLUENCE OF EGR ON KNOCK OCCURRENCE IN SI ENGINES, Combustion Science and Technology Volume 178, 2006 - Issue 9, Pages 1613-1647

[17]	Nobuyuki Kawahara, Eiji Tomita, Visualization of auto-ignition and pressure wave during knocking in a hydrogen spark-ignition engine, International Journal of Hydrogen Energy Volume 34, Issue 7, April 2009, Pages 3156-3163

[18]	Yunliang Qi, Zhi Wang, Jianxin Wang, Xin He, Effects of thermodynamic conditions on the end gas combustion mode associated with engine knock, Combustion and Flame Volume 162, Issue 11, November 2015, Pages 4119-4128

[19]	Jiaying Pan, Gequn Shu, Peng Zhao, Haiqiao Wei, Zheng Chen, Interactions of flame propagation, auto-ignition and pressure wave during knocking combustion, Combustion and Flame Volume 164, February 2016, Pages 319-328

[20]	Hao Yu, Zheng Chen, End-gas auto-ignition and detonation development in a closed chamber, Combustion and Flame Volume 162, Issue 11, November 2015, Pages 4102-4111

[21]	H. Sitaraman, S. Yellapantula, M.T.H. de Frahan, B. Perry, J. Rood, R. Grout, M. Day Adaptive mesh-based combustion simulations of direct fuel injection effects in a supersonic cavity flame holder, Combust. Flame 232 (2021), p. 111531

[22]	E.F. Toro, M. Spruce, W. Speares, Restoration of the contact surface in the HLL-Riemann solver, Shock Waves 4 (1994) 25–34.

[23]	P. Colella, P.R. Woodward, The Piecewise Parabolic Method (PPM) for gas-dynamical simulations, J Comput. Phys. 54 (1984) 174–201.

[24]	M. Emmett, E. Motheau, W. Zhang, M. Minion, J.B. Bell, A fourth-order adaptive mesh refinement algorithm for the multicomponent, reacting compressible Navier–Stokes equations, Combust. Theory and Modelling 23 (2019) 592–625.

[25]	A. Dutt, L. Greengard, V. Rokhlin, Spectral deferred correction methods for ordinary differential equations, BIT Numer. Math. 40 (2000) 241–266.

[26]	Y. Morii, E. Shima, Optimization of a one-parameter family of integration formulae for solving stiff chemical-kinetic ODEs, Sci Rep 10 (2020) 21330.

[27]	Y. Sakai, A. Miyoshi, Development of Reduced Chemical Kinetics Mechanism of Gasoline Surrogate Fuel [in Japanese], Proc. 28th Internal Combust. Engines Symp. (2017) 20178019.

[28]	Y. Sakai, A. Miyoshi, Development of Reduced Chemical Kinetics Mechanism of Gasoline Surrogate Fuel with Oxygenated Compounds [in Japanese], Proc. 29th Internal Combust. Engines Symp. (2018) 20183155.

[29]	A. Miyoshi, Y. Sakai, Gasoline surrogate detailed kinetic model revision 2.0 (SIP-Gd2.0) and reduced reaction mechanism revision 2.0 (SIP-Gr2.0), http://sip.st.keio.ac.jp/en/topics-en/gasolinesurrogate-detailed-kinetic-model-revision-1-0-sip-gd1-0-and-reduced-reaction-mechanism-revision1-0-sip-gr1-0-2/ (accessed Jan 13, 2023).

[30]	SIP, Innovative Combustion Technology, https://www.jst.go.jp/sip/k01.html (accessed Jan 13, 2023).

[31]	A. Miyoshi, Y. Sakai, Construction of a Detailed Kinetic Model for Gasoline Surrogate Mixtures, Transactions of Society of Automotive Engineers of Japan 48 (2017) 1021–1026.

[32]	P. Sela, Y. Sakai, H.S. Choi, J. Herzler, M. Fikri, C. Schulz, S. Peukert, High-Temperature Unimolecular Decomposition of Diethyl Ether: Shock-Tube and Theory Studies, J. Phys. Chem. A 123 (2019) 6813–6827.

[33]	P. Sela, S. Peukert, J. Herzler, C. Schulz, M. Fikri, Shock-tube study of the decomposition of octamethylcyclotetrasiloxane and hexamethylcyclotrisiloxane, Z. Phys. Chem. 234 (2020) 1395–1426.

[34]	M.J. McNenly, R.A. Whitesides, D.L. Flowers, Faster solvers for large kinetic mechanisms using adaptive preconditioners, Proc. Combust. Inst. 35 (2015) 581–587.

[35]	K. Kono, R. Shinya, S. Tanaka, Y. Nagano, T. Kitagawa, Study on knock phenomena during flame propagation in a constant volume vessel, 55th Symposium on Combustion, 2017.

[36]	Y. Nagano, T. Kitagawa, Observation of flame propagation and knock in a constant volume vessel, 25th International Colloquium on the Dynamics of Explosions and Reactive Systems, 2015.

[37]	Y. Ju, W. Sun, M.P. Burke, X. Gou, Z. Chen, Multi-timescale modelling of ignition and flame regimes of n-heptane-air mixtures near spark assisted homogeneous charge compression ignition conditions, Proc. Combust. Inst. 33 (2011)

[38]	 H. Terashima, M. Koshi, Mechanisms of strong pressure wave generation in end-gas auto-ignition during knocking combustion, Combust. Flame 162 (2015) 1944–1956.

[39]	Youhi Morii, Ajit K. Dubey, Hisashi Nakamura, Kaoru Maruta, Two-dimensional laboratory-scale DNS for knocking experiment using n-heptane at engine-like condition.

[40]	Ajit Kumar Dubey, Youhi Morii, Hisashi Nakamura, Kaoru Maruta, 2-D DNS study of Autoignition Modes in n-Heptane Combustion, 13th Asia-Pacific Conference on Combustion 2021 ADNEC, Abu Dhabi – UAE, 4 – 9 December 2021

[41]	Ajit Kumar Dubey, Youhi Morii, Hisashi Nakamura, and Kaoru Maruta, Autoignition modes for n-heptane combustion at high temperature and pressure: A 2-D DNS study

[42]	Jiaying Pan, Haiqiao Wei, Gequn Shu, Rui Chen, Effect of pressure wave disturbance on auto-ignition mode transition and knocking intensity under enclosed conditions

Appendix

Appendix-A

PeleC Inputs

# ------------------  INPUTS TO MAIN PROGRAM  -------------------

max_step = 150000

stop_time =  1.0

#amr.restart = chk107000

# PROBLEM SIZE & GEOMETRY

geometry.is_periodic = 0 0 0

geometry.coord_sys   = 0  # 0 => cart, 1 => RZ  2=>spherical

geometry.prob_lo     =  0.0 0.0 0.0

geometry.prob_hi     =  8.0 1.4 1.4

amr.n_cell           = 2560 448 224

# >>>>>>>>>>>>>  BC KEYWORDS <<<<<<<<<<<<<<<<<<<<<<

# Interior, UserBC, Symmetry, SlipWall, NoSlipWall

# >>>>>>>>>>>>>  BC KEYWORDS <<<<<<<<<<<<<<<<<<<<<<

pelec.lo_bc       = "NoSlipWall"   "NoSlipWall"   "NoSlipWall"

pelec.hi_bc       = "NoSlipWall"   "NoSlipWall"   "NoSlipWall"

# WHICH PHYSICS

pelec.do_hydro = 1

pelec.do_mol = 0

pelec.sdc_iters = 2

pelec.diffuse_vel = 1

pelec.diffuse_temp = 1

pelec.diffuse_spec = 1

pelec.diffuse_enth = 1

pelec.do_react = 1

pelec.chem_integrator=1

pelec.riemann_solver = 2

pelec.hybrid_riemann = 1

pelec.ppm_type = 1

# TIME STEP CONTROL

pelec.cfl            = 0.4     # cfl number for hyperbolic system

pelec.init_shrink    = 1.0     # scale back initial timestep

pelec.change_max     = 1.05    # scale back initial timestep

pelec.dt_cutoff      = 5.e-20  # level 0 timestep below which we halt

# DIAGNOSTICS & VERBOSITY

pelec.sum_interval   = 100     # timesteps between computing mass

pelec.v              = 1       # verbosity in Castro.cpp

amr.v                = 1       # verbosity in Amr.cpp

amr.data_log         = datlog

# REFINEMENT / REGRIDDING

amr.max_level       = 3       # maximum level number allowed

amr.ref_ratio       = 2 2 2 2 # refinement ratio

amr.regrid_int      = 2 2 2 2 # how often to regrid

amr.blocking_factor = 8       # block factor in grid generation

amr.max_grid_size   = 64

amr.n_error_buf     = 2 2 2 2 # number of buffer cells in error est

tagging.max_tempgrad_lev = 10

tagging.tempgrad = 10.0

# CHECKPOINT FILES

amr.checkpoint_files_output = 1

amr.check_file      = chk        # root name of checkpoint file

amr.check_int       = 500        # number of timesteps between checkpoints

# PLOTFILES

amr.plot_files_output = 1

amr.plot_file       = plt        # root name of plotfile

amr.plot_int        = 50        # number of timesteps between plotfiles

amr.plot_vars  =  density Temp

amr.derive_plot_vars = x_velocity y_velocity z_velocity magvel magvort pressure

pelec.plot_massfrac = 1

pelec.plot_rhoy = 0

# PROBLEM PARAMETERS

prob.MechFile = ./mechanism/mechanism.inp

prob.IgkR = 0.05

prob.IgkX0 = 0.25

prob.IgkY0 = 0.70

prob.IgkZ0 = 0.70

prob.Fname = NC7H16

prob.Oname = O2

prob.Iname = AR

prob.PresH = 3343725.0

prob.TempH = 1400.0

prob.FmoleH =  1.000

prob.OmoleH = 11.000

prob.ImoleH = 41.382

prob.PresL = 3343725.0

prob.TempL = 1150.0

prob.FmoleL =  1.000

prob.OmoleL = 11.000

prob.ImoleL = 41.382

# Problem setup

pelec.eb_boundary_T = 300.

pelec.eb_isothermal = 1

eb2.geom_type = box

eb2.box_lo = 1.0 0.0 0.0

eb2.box_hi = 1.4 0.1 0.1

eb2.box_has_fluid_inside = 0

ebd.boundary_grad_stencil_type=0

Appendix-B

i=135800

while (i <= 150000):

if i == 31700:

i+=100

continue

DeleteAllPlots()

x = str(i)

while(len(x) < 5):

x="0"+x

OpenDatabase("localhost:I:\\1000K\\work1000K\\plt" + x + "\\Header", 0)

# OpenDatabase("localhost:I:\\1000K\\work1000K\\plt\\Header", 0)

# MAINTENANCE ISSUE: SetSuppressMessagesRPC is not handled in Logging.C. Please contact a VisIt developer.

# SaveSession("C:\\Users\\Dr. Siladitya Pal\\Documents\\VisIt\\crash_recovery.18956.session")

# MAINTENANCE ISSUE: SetSuppressMessagesRPC is not handled in Logging.C. Please contact a VisIt developer.

DefineScalarExpression("operators/ConnectedComponents/Mesh", "cell_constant(<Mesh>, 0.)")

DefineCurveExpression("operators/DataBinning/1D/Mesh", "cell_constant(<Mesh>, 0)")

DefineScalarExpression("operators/DataBinning/2D/Mesh", "cell_constant(<Mesh>, 0)")

DefineScalarExpression("operators/DataBinning/3D/Mesh", "cell_constant(<Mesh>, 0)")

DefineScalarExpression("operators/Flux/Mesh", "cell_constant(<Mesh>, 0.)")

DefineScalarExpression("operators/IntegralCurve/_velocity", "cell_constant(<_velocity>, 0.)")

DefineScalarExpression("operators/LCS/_velocity", "cell_constant(<_velocity>, 0.)")

DefineVectorExpression("operators/LCS/_velocity", "cell_constant(<_velocity>, 0.)")

DefineCurveExpression("operators/Lagrangian/_velocity", "cell_constant(<_velocity>, 0.)")

DefineScalarExpression("operators/LimitCycle/_velocity", "cell_constant(<_velocity>, 0.)")

DefineCurveExpression("operators/LimitCycle/_velocity", "cell_constant(<_velocity>, 0.)")

DefineCurveExpression("operators/Lineout/Temp", "cell_constant(<Temp>, 0.)")

DefineCurveExpression("operators/Lineout/Y_lp_OH_rp_", "cell_constant(<Y_lp_OH_rp_>, 0.)")

DefineCurveExpression("operators/Lineout/density", "cell_constant(<density>, 0.)")

DefineCurveExpression("operators/Lineout/pressure", "cell_constant(<pressure>, 0.)")

DefineCurveExpression("operators/Lineout/x_velocity", "cell_constant(<x_velocity>, 0.)")

DefineCurveExpression("operators/Lineout/y_velocity", "cell_constant(<y_velocity>, 0.)")

DefineCurveExpression("operators/Lineout/z_velocity", "cell_constant(<z_velocity>, 0.)")

DefineScalarExpression("operators/ModelFit/distance", "point_constant(<Mesh>, 0)")

DefineScalarExpression("operators/ModelFit/model", "point_constant(<Mesh>, 0)")

DefineScalarExpression("operators/Poincare/_velocity", "cell_constant(<_velocity>, 0.)")

DefineScalarExpression("operators/StatisticalTrends/Mean/Temp", "cell_constant(<Temp>, 0.)")

DefineScalarExpression("operators/StatisticalTrends/Residuals/Y_lp_OH_rp_", "cell_constant(<Y_lp_OH_rp_>, 0.)")

DefineScalarExpression("operators/StatisticalTrends/Residuals/density", "cell_constant(<density>, 0.)")

DefineScalarExpression("operators/StatisticalTrends/Residuals/pressure", "cell_constant(<pressure>, 0.)")

DefineScalarExpression("operators/StatisticalTrends/Residuals/x_velocity", "cell_constant(<x_velocity>, 0.)")

DefineScalarExpression("operators/StatisticalTrends/Residuals/y_velocity", "cell_constant(<y_velocity>, 0.)")

DefineScalarExpression("operators/StatisticalTrends/Residuals/z_velocity", "cell_constant(<z_velocity>, 0.)")

DefineScalarExpression("operators/StatisticalTrends/Slope/Temp", "cell_constant(<Temp>, 0.)")

DefineScalarExpression("operators/StatisticalTrends/Variance/Y_lp_OH_rp_", "cell_constant(<Y_lp_OH_rp_>, 0.)")

DefineScalarExpression("operators/StatisticalTrends/Variance/density", "cell_constant(<density>, 0.)")

DefineScalarExpression("operators/StatisticalTrends/Variance/pressure", "cell_constant(<pressure>, 0.)")

DefineScalarExpression("operators/StatisticalTrends/Variance/x_velocity", "cell_constant(<x_velocity>, 0.)")

DefineScalarExpression("operators/StatisticalTrends/Variance/y_velocity", "cell_constant(<y_velocity>, 0.)")

DefineScalarExpression("operators/StatisticalTrends/Variance/z_velocity", "cell_constant(<z_velocity>, 0.)")

DefineVectorExpression("operators/SurfaceNormal/Mesh", "cell_constant(<Mesh>, 0.)")

DefineScalarExpression("dgrad", "magnitude(gradient(density))")

AddPlot("Pseudocolor", "dgrad", 1, 1)

DrawPlots()

PseudocolorAtts = PseudocolorAttributes()

PseudocolorAtts.scaling = PseudocolorAtts.Linear  # Linear, Log, Skew

PseudocolorAtts.skewFactor = 1

PseudocolorAtts.limitsMode = PseudocolorAtts.OriginalData  # OriginalData, ActualData

PseudocolorAtts.minFlag = 0

PseudocolorAtts.min = 0

PseudocolorAtts.useBelowMinColor = 0

PseudocolorAtts.belowMinColor = (0, 0, 0, 255)

PseudocolorAtts.maxFlag = 1

PseudocolorAtts.max = 5e-05

PseudocolorAtts.useAboveMaxColor = 0

PseudocolorAtts.aboveMaxColor = (0, 0, 0, 255)

PseudocolorAtts.centering = PseudocolorAtts.Natural  # Natural, Nodal, Zonal

PseudocolorAtts.colorTableName = "gray"

PseudocolorAtts.invertColorTable = 0

PseudocolorAtts.opacityType = PseudocolorAtts.FullyOpaque  # ColorTable, FullyOpaque, Constant, Ramp, VariableRange

PseudocolorAtts.opacityVariable = ""

PseudocolorAtts.opacity = 1

PseudocolorAtts.opacityVarMin = 0

PseudocolorAtts.opacityVarMax = 1

PseudocolorAtts.opacityVarMinFlag = 0

PseudocolorAtts.opacityVarMaxFlag = 0

PseudocolorAtts.pointSize = 0.05

PseudocolorAtts.pointType = PseudocolorAtts.Point  # Box, Axis, Icosahedron, Octahedron, Tetrahedron, SphereGeometry, Point, Sphere

PseudocolorAtts.pointSizeVarEnabled = 0

PseudocolorAtts.pointSizeVar = "default"

PseudocolorAtts.pointSizePixels = 2

PseudocolorAtts.lineType = PseudocolorAtts.Line  # Line, Tube, Ribbon

PseudocolorAtts.lineWidth = 0

PseudocolorAtts.tubeResolution = 10

PseudocolorAtts.tubeRadiusSizeType = PseudocolorAtts.FractionOfBBox  # Absolute, FractionOfBBox

PseudocolorAtts.tubeRadiusAbsolute = 0.125

PseudocolorAtts.tubeRadiusBBox = 0.005

PseudocolorAtts.tubeRadiusVarEnabled = 0

PseudocolorAtts.tubeRadiusVar = ""

PseudocolorAtts.tubeRadiusVarRatio = 10

PseudocolorAtts.tailStyle = PseudocolorAtts.NONE  # NONE, Spheres, Cones

PseudocolorAtts.headStyle = PseudocolorAtts.NONE  # NONE, Spheres, Cones

PseudocolorAtts.endPointRadiusSizeType = PseudocolorAtts.FractionOfBBox  # Absolute, FractionOfBBox

PseudocolorAtts.endPointRadiusAbsolute = 0.125

PseudocolorAtts.endPointRadiusBBox = 0.05

PseudocolorAtts.endPointResolution = 10

PseudocolorAtts.endPointRatio = 5

PseudocolorAtts.endPointRadiusVarEnabled = 0

PseudocolorAtts.endPointRadiusVar = ""

PseudocolorAtts.endPointRadiusVarRatio = 10

PseudocolorAtts.renderSurfaces = 1

PseudocolorAtts.renderWireframe = 0

PseudocolorAtts.renderPoints = 0

PseudocolorAtts.smoothingLevel = 0

PseudocolorAtts.legendFlag = 1

PseudocolorAtts.lightingFlag = 1

PseudocolorAtts.wireframeColor = (0, 0, 0, 0)

PseudocolorAtts.pointColor = (0, 0, 0, 0)

SetPlotOptions(PseudocolorAtts)

# Begin spontaneous state

# View2DAtts = View2DAttributes()

# View2DAtts.windowCoords = (0, 8, 0, 1.4)

# View2DAtts.viewportCoords = (0.2, 0.95, 0.15, 0.95)

# View2DAtts.fullFrameActivationMode = View2DAtts.Auto  # On, Off, Auto

# View2DAtts.fullFrameAutoThreshold = 100

# View2DAtts.xScale = View2DAtts.LINEAR  # LINEAR, LOG

# View2DAtts.yScale = View2DAtts.LINEAR  # LINEAR, LOG

# View2DAtts.windowValid = 1

# SetView2D(View2DAtts)

# End spontaneous state

SaveWindowAtts = SaveWindowAttributes()

SaveWindowAtts.outputToCurrentDirectory = 0

SaveWindowAtts.outputDirectory = r"C:\Users\Dr. Siladitya Pal\Documents\VisIt\My images"

SaveWindowAtts.fileName = "visit"

SaveWindowAtts.family = 1

SaveWindowAtts.format = SaveWindowAtts.PNG  # BMP, CURVE, JPEG, OBJ, PNG, POSTSCRIPT, POVRAY, PPM, RGB, STL, TIFF, ULTRA, VTK, PLY, EXR

SaveWindowAtts.width = 824

SaveWindowAtts.height = 824

SaveWindowAtts.screenCapture = 0

SaveWindowAtts.saveTiled = 0

SaveWindowAtts.quality = 80

SaveWindowAtts.progressive = 0

SaveWindowAtts.binary = 0

SaveWindowAtts.stereo = 0

SaveWindowAtts.compression = SaveWindowAtts.NONE  # NONE, PackBits, Jpeg, Deflate, LZW

SaveWindowAtts.forceMerge = 0

SaveWindowAtts.resConstraint = SaveWindowAtts.EqualWidthHeight  # NoConstraint, EqualWidthHeight, ScreenProportions

SaveWindowAtts.pixelData = 1

SaveWindowAtts.advancedMultiWindowSave = 0

SaveWindowAtts.subWindowAtts.win1.position = (0, 0)

SaveWindowAtts.subWindowAtts.win1.size = (128, 128)

SaveWindowAtts.subWindowAtts.win1.layer = 0

SaveWindowAtts.subWindowAtts.win1.transparency = 0

SaveWindowAtts.subWindowAtts.win1.omitWindow = 0

SaveWindowAtts.subWindowAtts.win2.position = (0, 0)

SaveWindowAtts.subWindowAtts.win2.size = (128, 128)

SaveWindowAtts.subWindowAtts.win2.layer = 0

SaveWindowAtts.subWindowAtts.win2.transparency = 0

SaveWindowAtts.subWindowAtts.win2.omitWindow = 0

SaveWindowAtts.subWindowAtts.win3.position = (0, 0)

SaveWindowAtts.subWindowAtts.win3.size = (128, 128)

SaveWindowAtts.subWindowAtts.win3.layer = 0

SaveWindowAtts.subWindowAtts.win3.transparency = 0

SaveWindowAtts.subWindowAtts.win3.omitWindow = 0

SaveWindowAtts.subWindowAtts.win4.position = (0, 0)

SaveWindowAtts.subWindowAtts.win4.size = (128, 128)

SaveWindowAtts.subWindowAtts.win4.layer = 0

SaveWindowAtts.subWindowAtts.win4.transparency = 0

SaveWindowAtts.subWindowAtts.win4.omitWindow = 0

SaveWindowAtts.subWindowAtts.win5.position = (0, 0)

SaveWindowAtts.subWindowAtts.win5.size = (128, 128)

SaveWindowAtts.subWindowAtts.win5.layer = 0

SaveWindowAtts.subWindowAtts.win5.transparency = 0

SaveWindowAtts.subWindowAtts.win5.omitWindow = 0

SaveWindowAtts.subWindowAtts.win6.position = (0, 0)

SaveWindowAtts.subWindowAtts.win6.size = (128, 128)

SaveWindowAtts.subWindowAtts.win6.layer = 0

SaveWindowAtts.subWindowAtts.win6.transparency = 0

SaveWindowAtts.subWindowAtts.win6.omitWindow = 0

SaveWindowAtts.subWindowAtts.win7.position = (0, 0)

SaveWindowAtts.subWindowAtts.win7.size = (128, 128)

SaveWindowAtts.subWindowAtts.win7.layer = 0

SaveWindowAtts.subWindowAtts.win7.transparency = 0

SaveWindowAtts.subWindowAtts.win7.omitWindow = 0

SaveWindowAtts.subWindowAtts.win8.position = (0, 0)

SaveWindowAtts.subWindowAtts.win8.size = (128, 128)

SaveWindowAtts.subWindowAtts.win8.layer = 0

SaveWindowAtts.subWindowAtts.win8.transparency = 0

SaveWindowAtts.subWindowAtts.win8.omitWindow = 0

SaveWindowAtts.subWindowAtts.win9.position = (0, 0)

SaveWindowAtts.subWindowAtts.win9.size = (128, 128)

SaveWindowAtts.subWindowAtts.win9.layer = 0

SaveWindowAtts.subWindowAtts.win9.transparency = 0

SaveWindowAtts.subWindowAtts.win9.omitWindow = 0

SaveWindowAtts.subWindowAtts.win10.position = (0, 0)

SaveWindowAtts.subWindowAtts.win10.size = (128, 128)

SaveWindowAtts.subWindowAtts.win10.layer = 0

SaveWindowAtts.subWindowAtts.win10.transparency = 0

SaveWindowAtts.subWindowAtts.win10.omitWindow = 0

SaveWindowAtts.subWindowAtts.win11.position = (0, 0)

SaveWindowAtts.subWindowAtts.win11.size = (128, 128)

SaveWindowAtts.subWindowAtts.win11.layer = 0

SaveWindowAtts.subWindowAtts.win11.transparency = 0

SaveWindowAtts.subWindowAtts.win11.omitWindow = 0

SaveWindowAtts.subWindowAtts.win12.position = (0, 0)

SaveWindowAtts.subWindowAtts.win12.size = (128, 128)

SaveWindowAtts.subWindowAtts.win12.layer = 0

SaveWindowAtts.subWindowAtts.win12.transparency = 0

SaveWindowAtts.subWindowAtts.win12.omitWindow = 0

SaveWindowAtts.subWindowAtts.win13.position = (0, 0)

SaveWindowAtts.subWindowAtts.win13.size = (128, 128)

SaveWindowAtts.subWindowAtts.win13.layer = 0

SaveWindowAtts.subWindowAtts.win13.transparency = 0

SaveWindowAtts.subWindowAtts.win13.omitWindow = 0

SaveWindowAtts.subWindowAtts.win14.position = (0, 0)

SaveWindowAtts.subWindowAtts.win14.size = (128, 128)

SaveWindowAtts.subWindowAtts.win14.layer = 0

SaveWindowAtts.subWindowAtts.win14.transparency = 0

SaveWindowAtts.subWindowAtts.win14.omitWindow = 0

SaveWindowAtts.subWindowAtts.win15.position = (0, 0)

SaveWindowAtts.subWindowAtts.win15.size = (128, 128)

SaveWindowAtts.subWindowAtts.win15.layer = 0

SaveWindowAtts.subWindowAtts.win15.transparency = 0

SaveWindowAtts.subWindowAtts.win15.omitWindow = 0

SaveWindowAtts.subWindowAtts.win16.position = (0, 0)

SaveWindowAtts.subWindowAtts.win16.size = (128, 128)

SaveWindowAtts.subWindowAtts.win16.layer = 0

SaveWindowAtts.subWindowAtts.win16.transparency = 0

SaveWindowAtts.subWindowAtts.win16.omitWindow = 0

SaveWindowAtts.opts.types = ()

SaveWindowAtts.opts.help = ""

SetSaveWindowAttributes(SaveWindowAtts)

SaveWindow()

i+=100

VisIT Post-Processing Automation MACRO

VisIT Python Macro

Appendix-C Gasoline PRF Reduced Mechanism 2.0 (#190128a)

!

! ===== gasoline PRF reduced mechanism 2.0 (#190128a) =====

!       with ethanol

!

! Fuel: nC7H16(n-heptane)

!       iC8H18(isooctane, 2,2,4-trimethylpentane)

!       C2H5OH(ethanol)

!

!-------------------------------------------------------------------------------

ELEMENTS

H              O              C              N

END

!-------------------------------------------------------------------------------

SPECIES

!

!n-heptane mechanism

!

nC7H16         nC7H15         nC7H15OO       nC7H14OOH      nHOOC7H14OO

nC7KET         nC5H9O         nC5H11         nC5H10         nC5H9

nC7H15O        nC7H15OOH

!

!isooctane mechanism

!

iC8H18         iC8H17         iC8H17OO       iC8H16OOH      iHOOC8H16OO

iC8KET         iC7H13O        iC7H14         iC7H13         iC4H9

iC4H8          iC4H7          iC8H17O        iC8H17OOH

!

!ethanol mechanism

!

C2H5OH         sC2H4OH        CH2OH

!

!base mechanism

!

N2             O              O2             H              OH

HO2            H2             H2O            H2O2           CO

CO2            CH             HCO            CH2            CH2O

CH3            CH3O           CH3OO          CH4            CH3OOH

HCCO           C2H2           CH2CO          C2H3           CH2CHO

CH3CO          CH3CO2         CH3C(O)OO      C2H4           CH3CHO

CH3C(O)OOH     C2H5           pC2H4OH        C2H5O          C2H5OO

C2H6           C2H5OOH        aC3H5          C3H6           iC3H7

END

!-------------------------------------------------------------------------------

REACTIONS

!

!n-heptane mechanism

!

nC7H16=nC5H11+C2H5                         1.014E+25   -2.254   88858.0 !

nC7H16+O2=nC7H15+HO2                       2.704E+13    0.162   48236.7 !

nC7H16+OH=nC7H15+H2O                       5.738E+06    2.034    -797.9 !

nC7H16+HO2=nC7H15+H2O2                     9.496E+12    0.177   17684.1 !

nC7H16+H=nC7H15+H2                         2.240E+05    2.859    3844.8 !

nC7H16+CH3=nC7H15+CH4                      1.030E-03    4.704    3475.1 !

nC7H16+O=nC7H15+OH                         1.927E+05    2.765    2170.5 !

nC7H15+O2=nC7H15OO                         3.480E+14   -0.816    -530.0 !

nC7H15=nC5H10+C2H5                         1.000E+13    0.000   28000.0 !

nC7H15OO=nC7H14OOH                         5.100E+11    0.000   20900.0 !

nC7H14OOH+O2=nHOOC7H14OO                   3.480E+14   -0.816    -530.0 !

nC7H14OOH=>nC5H10+CH3CHO+OH                3.300E+11    0.000   14000.0 !

nHOOC7H14OO=>nC7KET+OH                     1.000E+11    0.000   20000.0 !

nC7KET=>CH3CHO+nC5H9O+OH                   3.000E+15    0.000   41600.0 !

nC5H9O=>iC3H7+CH2CO                        1.000E+11    0.000   28000.0 !

nC5H9O=>C3H6+CH2CHO                        1.000E+12    0.000   28000.0 !

nC5H11=>C2H4+iC3H7                         2.961E+19   -2.041   30921.8 !

nC5H10+OH=nC5H9+H2O                        2.000E+12    0.000    2100.0 !

nC5H10+H=nC5H9+H2                          5.000E+11    0.000     900.0 !

nC5H10+CH3=nC5H9+CH4                       7.000E+10    0.000    8500.0 !

nC5H10+O=>iC3H7+CH2CHO                     9.000E+13    0.000    1710.0 !

nC5H9+O2=>CH3CHO+C2H4+HCO                  3.000E+12    0.000   18800.0 !

!

!isooctane mechanism

!

iC8H18=iC4H9+iC4H9                         7.708E+26   -3.095   87054.7 !

iC8H18+O2=iC8H17+HO2                       1.479E+11    0.789   46956.6 !

iC8H18+OH=iC8H17+H2O                       1.056E+07    1.990    -151.8 !

iC8H18+HO2=iC8H17+H2O2                     1.348E+11    0.692   16979.3 !

iC8H18+H=iC8H17+H2                         3.768E+02    3.656    2769.0 !

iC8H18+CH3=iC8H17+CH4                      9.083E-08    5.968    2884.7 !

iC8H17+O2=iC8H17OO                         3.480E+14   -0.816    -530.0 !

iC8H17=iC7H14+CH3                          9.000E+10    0.000   28000.0 !

iC8H17=iC4H8+iC4H9                         3.000E+12    0.000   28000.0 !

iC8H17OO=iC8H16OOH                         7.840E+11    0.000   22570.0 !

iC8H16OOH+O2=iHOOC8H16OO                   6.890E+16   -1.627     200.0 !

iC8H16OOH=>iC7H14+CH2O+OH                  2.000E+12    0.000   17140.0 !

iHOOC8H16OO=iC8KET+OH                      1.300E+12    0.000   27000.0 !

iC8KET=>CH2O+iC7H13O+OH                    3.000E+15    0.000   41600.0 !

iC7H13O=>iC3H7+C2H4+CH2CO                  3.000E+13    0.000   28000.0 !

iC7H14+OH=iC7H13+H2O                       4.000E+13    0.000    2100.0 !

iC7H14+HO2=iC7H13+H2O2                     9.000E+08    1.485   15419.4 !

iC7H14+O=iC7H13O+H                         1.500E+04    1.880     183.0 !

iC7H13+O2=>CH3CHO+iC4H8+HCO                1.000E+12    0.000   18800.0 !

iC4H9=iC4H8+H                              1.000E+14    0.000   37560.6 !

iC4H9+O2=iC4H8+HO2                         7.140E+15   -1.210   21050.0 !

iC4H8+OH=iC4H7+H2O                         1.000E+07    2.000    -298.0 !

iC4H8+HO2=iC4H7+H2O2                       9.000E+08    1.485   15419.4 !

iC4H8=iC4H7+H                              3.000E+32   -4.795   97024.0 !

iC4H7=>C2H4+C2H3                           3.000E+47   -9.740   74360.0 !

iC4H7+O2=>C2H4+CH2O+HCO                    7.140E+15   -1.210   21050.0 !

iC4H7+HO2=>CH3CHO+C2H3+OH                  2.000E+13    0.000    1000.0 !

!

!ethanol mechanism

!

C2H5OH(+M)=C2H4+H2O(+M)                    2.726E+05    2.3169  62954.0 !

LOW/6.090E+90  -21.0692  87325.0/!

TROE / 0.9 50000 3500 /

H2O/5/

C2H5OH(+M)=CH2OH+CH3(+M)                   1.916E+29   -3.7818  92416.0 !

LOW /  6.440E+58 -11.2274  99333.0 /

TROE / 0.7 200 350 3500 /

H2/2/ H2O/5/ CO/2/ CO2/3/

C2H5OH+O2=pC2H4OH+HO2                      8.910E+01    3.124   45860.0 !

C2H5OH+O2=sC2H4OH+HO2                      4.860E+01    3.330   37020.0 !

C2H5OH+O2=C2H5O+HO2                        6.630E+03    2.892   49190.0 !

C2H5OH+OH=pC2H4OH+H2O                      5.130E+01    3.203   -1760.0 !

C2H5OH+OH=sC2H4OH+H2O                      3.555E+04    2.526   -1900.0 !

C2H5OH+OH=C2H5O+H2O                        4.290E+02    2.906     650.0 !

C2H5OH+HO2=sC2H4OH+H2O2                    2.220E-02    4.161    7060.0 !

C2H5OH+HO2=pC2H4OH+H2O2                    7.350E-03    4.021   14280.0 !

C2H5OH+HO2=C2H5O+H2O2                      1.450E-01    3.822   18430.0 !

C2H5OH+H=pC2H4OH+H2                        1.230E+07    1.800    5098.0 !

C2H5OH+H=sC2H4OH+H2                        2.580E+07    1.650    2827.0 !

C2H5OH+H=C2H5O+H2                          1.500E+07    1.600    3038.0 !

C2H5OH+CH3OO=C2H5O+CH3OOH                  1.260E+12    0.000   20440.0 !

C2H5OH+CH3OO=pC2H4OH+CH3OOH                8.400E+12    0.000   20440.0 !

C2H5OH+CH3OO=sC2H4OH+CH3OOH                1.400E+13    0.000   16340.0 !

sC2H4OH+O2=CH3CHO+HO2                      3.810E+06    2.000    1641.0 !

CH2OH+O2=CH2O+HO2                          3.810E+06    2.000    1641.0 !

!

!cross (RH+R'=R+R'OOH, R+R'OO=RO+R'O, ROO+R'OO=RO+R'O+O2, RO and ROOH decomp)

!

!PRF+ethanol

nC7H16+iC8H17OO=nC7H15+iC8H17OOH           9.496E+12    0.177   17684.1 !

iC8H18+nC7H15OO=iC8H17+nC7H15OOH           1.348E+11    0.692   16979.3 !

C2H5OH+nC7H15OO=C2H5O+nC7H15OOH            1.260E+12    0.000   20440.0 !

C2H5OH+nC7H15OO=pC2H4OH+nC7H15OOH          8.400E+12    0.000   20440.0 !

C2H5OH+nC7H15OO=sC2H4OH+nC7H15OOH          1.400E+13    0.000   16340.0 !

C2H5OH+iC8H17OO=C2H5O+iC8H17OOH            1.260E+12    0.000   20440.0 !

C2H5OH+iC8H17OO=pC2H4OH+iC8H17OOH          8.400E+12    0.000   20440.0 !

C2H5OH+iC8H17OO=sC2H4OH+iC8H17OOH          1.400E+13    0.000   16340.0 !

nC7H15+iC8H17OO=>nC7H15O+iC8H17O           7.000E+12    0.000   -1000.0 !

iC8H17+nC7H15OO=>iC8H17O+nC7H15O           7.000E+12    0.000   -1000.0 !

nC7H15OO+iC8H17OO=>nC7H15O+iC8H17O+O2      1.400E+16   -1.610    1860.0 !

nC7H15O=>CH2O+nC5H10+CH3                   1.000E+11    0.000   11900.0 !

nC7H15OOH=nC7H15O+OH                       3.000E+15    0.000   41600.0 !

iC8H17O=>C2H4+CH3CHO+iC4H9                 1.000E+11    0.000   11900.0 !

iC8H17OOH=iC8H17O+OH                       3.000E+15    0.000   41600.0 !

!

!base mechanism

!

O2+H=OH+O                                  9.756E+13    0.000   14844.6

H2+O=OH+H                                  5.120E+04    2.670    6278.7

OH+OH=O+H2O                                1.510E+09    1.140     100.4

H2O+H=H2+OH                                4.520E+08    1.600   18422.6

H2O2+OH=H2O+HO2                            7.830E+12    0.000    1331.3

H+OH+M=H2O+M                               1.659E+23   -2.000       0.0

N2/0.133/ O2/0.133/ CO/0.25/ CO2/0.5/ H2O/0.85/

O2+H+M=HO2+M                               6.300E+18   -0.800       0.0

N2/0.223/ O2/0.133/ CO/0.25/ CO2/0.5/ H2O/0/

O2+H+H2O=HO2+H2O                           6.890E+15    0.000   -2086.5

O+HO2=O2+OH                                3.190E+13    0.000       0.0

H+HO2=OH+OH                                1.690E+14    0.000     874.8

H+HO2=H2+O2                                4.280E+13    0.000    1410.1

H+HO2=H2O+O                                3.010E+13    0.000    1720.8

OH+HO2=H2O+O2                              2.890E+13    0.000    -497.1

HO2+HO2=H2O2+O2                            4.220E+14    0.000   11983.7

duplicate

HO2+HO2=H2O2+O2                            1.320E+11    0.000   -1630.0

duplicate

OH+OH(+M)=H2O2(+M)                         7.230E+13   -0.370       0.0

LOW / 5.530E+19   -0.760   0.0 /

TROE / 1   1   1   1040 /

N2/0.4/ O2/0.4/ CO/0.75/ CO2/1.5/ H2O/6.5/

H2O2+H=OH+H2O                              1.020E+13    0.000    3577.9

H2O2+H=HO2+H2                              1.690E+12    0.000    3754.8

H+O+M=OH+M                                 3.540E+19   -1.000       0.0

N2/0.133/ O2/0.133/ CO/0.25/ CO2/0.5/ H2O/2.17/

O+O+M=O2+M                                 1.620E+14    0.000   -1787.8

N2/0.133/ O2/0.133/ CO/0.25/ CO2/0.5/ H2O/2.17/

H+H+M=H2+M                                 5.610E+18   -1.000       0.0

N2/0.133/ O2/0.133/ CO/0.25/ CO2/0.5/ H2O/2.17/ H2/0/

H2O2+O=OH+HO2                              6.620E+11    0.000    3974.7

CH3+H(+M)=CH4(+M)                          2.138E+15   -0.400       0.0

LOW / 3.310E+30   -4.000   2108.0 /

TROE / 0   1E-15   1E-15   40 /

H2/2/ H2O/5/ CO/2/ CO2/3/

CH4+H=CH3+H2                               1.727E+04   3.000    8224.0

REV /  6.610E+02   3.000    7744.0 /

CH4+OH=CH3+H2O                             1.930E+05   2.400    2106.0

REV /  4.820E+02   2.900   14860.0 /

CH4+O=CH3+OH                               2.130E+06   2.210    6480.0

REV /  3.557E+04   2.210    3920.0 /

HCO+OH=CO+H2O                              3.011E+13   0.000       0.0 ! AramcoMech2.0

CO+OH=CO2+H                                1.400E+05   1.950   -1347.0

REV /  1.568E+07   1.950   20990.0 /

HCO+M=H+CO+M                               1.860E+17  -1.000   17000.0

REV /  6.467E+13   0.000    -442.0 /

CH3+HO2=CH3O+OH                            1.100E+13   0.000       0.0

REV /  4.780E+14  -0.350   24550.0 /

CO+HO2=CO2+OH                              3.010E+13   0.000   23000.0

REV /  6.435E+15  -0.330   84610.0 /

CO+O(+M)=CO2(+M)                           1.800E+10   0.000    2384.0

LOW / 1.350E+24   -2.788   4191.0 /

H2/2.5/ H2O/12/ CO/1.9/ CO2/3.8/

CO+O2=CO2+O                                1.050E+12   0.      42540.0   ! Curran 2010

REV /  8.035E+15  -0.8     51230.0 / ! Curran 2010

HCO+H=CO+H2                                7.340E+13   0.000       0.0

REV /  4.813E+14   0.000   90000.0 /

HCO+O=CO+OH                                3.020E+13   0.000       0.0

REV /  8.697E+13   0.000   87900.0 /

CH2O+M=HCO+H+M                             6.283E+29  -3.570   93200.0

REV /  2.660E+24  -2.570     427.0 /

CH2O+OH=HCO+H2O                            3.430E+09   1.180    -447.0

REV /  1.186E+09   1.180   29380.0 /

CH2O+H=HCO+H2                              9.334E+08   1.500    2976.0

REV /  7.453E+07   1.500   17650.0 /

CH2O+O=HCO+OH                              4.160E+11   0.570    2762.0

REV /  1.459E+10   0.570   15340.0 /

CH3+OH=CH2O+H2                             2.250E+13   0.000    4300.0

REV /  6.756E+14   0.000   76030.0 /

CH3+O=CH2O+H                               8.000E+13   0.000       0.0

REV /  1.055E+15   0.000   69630.0 /

CH3+O2=CH3O+O                              1.995E+18  -1.570   29210.0

REV /  3.585E+18  -1.590   -1631.0 /

CH2O+CH3=HCO+CH4                           3.636E-06   5.420     998.0

REV /  7.584E-06   5.420   16150.0 /

HCO+CH3=CH4+CO                             2.650E+13   0.000       0.0 ! AramcoMech2.0

HCO+HCO=CH2O+CO                            1.800E+13   0.000       0.0 ! AramcoMech2.0

CH3O(+M)=CH2O+H(+M)                        5.450E+13   0.000   13500.0

LOW / 2.344E+25   -2.700   30600.0 /

HCO+HO2=CH2O+O2                            2.974E+10   0.330   -3861.0

REV /  2.050E+13   0.000   38950.0 /

CH3O+O2=CH2O+HO2                           5.500E+10   0.000    2424.0

REV /  1.318E+09   0.350   31390.0 /

CH3+HO2=CH4+O2                             3.600E+12   0.000       0.0

REV /  5.177E+15  -0.330   57960.0 /

HCO+O2=CO+HO2                              7.580E+12   0.000     410.0

REV /  9.029E+11   0.330   32930.0 /

CH4+HO2=CH3+H2O2                           3.420E+11   0.000   19290.0

REV /  3.365E+11  -0.330    2502.0 /

CH2O+HO2=HCO+H2O2                          5.820E-03   4.530    6557.0

REV /  1.194E-02   4.200    4921.0 /

CH2+O2=CO+H2O                              7.280E+19  -2.540    1809.0

REV /  8.508E+20  -2.540  179800.0 /

CH2+O2=HCO+OH                              1.290E+20  -3.300     284.0

REV /  5.310E+19  -3.300   73170.0 /

CH2+O=>CO+H+H                              5.000E+13   0.000       0.0

CH2+H=CH+H2                                1.000E+18  -1.560       0.0

REV /  7.026E+17  -1.560    2990.0 /

CH2+OH=CH+H2O                              1.130E+07   2.000    3000.0

REV /  3.437E+07   2.000   21150.0 /

CH2+O2=>CO2+H+H                            3.290E+21  -3.300    2868.0

CH+O2=HCO+O                                3.300E+13   0.000       0.0

REV /  4.402E+13   0.000   71990.0 /

CH2+O2=CH2O+O                              3.290E+21  -3.300    2868.0

REV /  3.862E+22  -3.300   63180.0 /

CH2+O2=CO2+H2                              1.010E+21  -3.300    1508.0

REV /  3.054E+23  -3.300  186700.0 /

CH2O+M=CO+H2+M                             1.826E+32  -4.420   87120.0

REV /  5.070E+27  -3.420   84350.0 /

HCO+O=CO2+H                                3.000E+13   0.000       0.0

REV /  9.677E+15   0.000  110200.0 /

CH3+M=CH2+H+M                              1.968E+16   0.000   92520.0

REV /  2.107E+11   1.000  -19620.0 /

CH3+H=CH2+H2                               9.000E+13   0.000   15100.0

REV /  1.818E+13   0.000   10400.0 /

CH3+OH=CH2+H2O                             3.000E+06   2.000    2500.0

REV /  2.623E+06   2.000   12960.0 /

CH3OO+M=CH3+O2+M                           4.343E+27  -3.420   30470.0

REV /  5.440E+25  -3.300       0.0 /

CH3OOH=CH3O+OH                             6.310E+14   0.000   42300.0

REV /  1.166E+11   0.600   -1771.0 /

CH3OO+CH2O=CH3OOH+HCO                      1.990E+12   0.000   11670.0

REV /  8.504E+12  -0.500    7009.0 /

CH4+CH3OO=CH3+CH3OOH                       1.810E+11   0.000   18480.0

REV /  3.708E+11  -0.500   -1327.0 /

CH3OO+CH3=CH3O+CH3O                        7.000E+12   0.000   -1000.0

REV /  2.971E+16  -0.930   28310.0 /

CH3OO+HO2=CH3OOH+O2                        1.750E+10   0.000   -3275.0

REV /  5.156E+13  -0.830   34880.0 /

CH3OO+CH3OO=>O2+CH3O+CH3O                  1.400E+16  -1.610    1860.0

CH3+O2=CH2O+OH                             7.470E+11   0.000   14250.0

REV /  7.778E+11   0.000   67770.0 /

C2H6+CH3=C2H5+CH4                          1.510E-07   6.000    6047.0

REV /  9.649E-10   6.560   10220.0 /

C2H4+O=CH3+HCO                             1.020E+07   1.880     179.0

REV /  2.851E+08   1.050   31770.0 /

H+C2H4(+M)=C2H5(+M)                        1.081E+12   0.450    1822.0

LOW / 1.112E+34   -5.000   4448.0 /

TROE / 1   1E-15   95   200 /

H2/2/ H2O/5/ CO/2/ CO2/3/

C2H6+H=C2H5+H2                             5.540E+02   3.500    5167.0

REV /  1.355E-01   4.060    8857.0 /

C2H5+O2=C2H4+HO2                           1.220E+30  -5.760   10100.0

REV /  1.259E+30  -5.630   22310.0 /

C2H6+OH=C2H5+H2O                           5.125E+06   2.060     855.0

REV /  1.010E+07   2.060   22980.0 /

C2H6+O=C2H5+OH                             1.130E+14   0.000    7850.0

REV /  2.080E+13   0.000   12720.0 /

CH3+CH3(+M)<=>C2H6(+M)                     2.277E+15  -0.690     174.9 ! AramcoMech2.0

LOW / 8.054E+31   -3.750E+00   9.816E+02 /

TROE / 0.000E+00   5.700E+02   1.000E+30   1.000E+30 /

H2O/5.0/ CO/2.0/ CO2/3.0/

C2H4(+M)=C2H2+H2(+M)                       1.800E+13   0.000   76000.0

LOW / 1.500E+15   0.000   55443.0 /

C2H3+H(+M)=C2H4(+M)                        6.100E+12   0.270     280.0

LOW / 9.800E+29   -3.860   3320.0 /

TROE / 0.782   208   2663   6095 /

C2H2+H(+M)=C2H3(+M)                        3.110E+11   0.580    2589.0

LOW / 2.254E+40   -7.269   6577.0 /

TROE / 1   1E-15   675   1E+15 /

H2/2/ H2O/5/ CO/2/ CO2/3/

C2H4+H=C2H3+H2                             8.420E-03   4.620    2583.0

REV /  5.723E-01   3.790    3233.0 /

C2H4+OH=C2H3+H2O                           1.800E+06   2.000    2500.    ! Curran2010

C2H2+O2=HCCO+OH                            2.000E+08   1.500   30100.0

REV /  2.232E+05   1.500   25400.0 /

C2H2+O=CH2+CO                              6.120E+06   2.000    1900.0

REV /  1.152E+06   2.000   52570.0 /

C2H2+O=HCCO+H                              1.430E+07   2.000    1900.0

REV /  2.021E+05   2.000   13310.0 /

C2H2+OH=CH2CO+H                            2.190E-04   4.500   -1000.0

REV /  2.161E-03   4.500   19670.0 /

CH2CO+H=CH3+CO                             1.100E+13   0.000    3400.0

REV /  2.400E+12   0.000   40200.0 /

CH2CO+O=CH2+CO2                            1.750E+12   0.000    1350.0

REV /  3.739E+12   0.000   53690.0 /

CH2CO(+M)=CH2+CO(+M)                       3.000E+14   0.000   70980.0

LOW /  3.600E+15   0.000   59270.0 /

CH2CO+O=HCCO+OH                            1.000E+13   0.000    8000.0

REV /  1.432E+10   0.000   -1255.0 /

CH2CO+OH=HCCO+H2O                          1.000E+13   0.000    2000.0

REV /  1.412E+11   0.000    9995.0 /

CH2CO+H=HCCO+H2                            2.000E+14   0.000    8000.0

REV /  6.522E+11   0.000     840.0 /

HCCO+OH=>H2+CO+CO                          1.000E+14   0.000       0.0 ! AramcoMech2.0

HCCO+O=>H+CO+CO                            8.000E+13   0.000       0.0 ! AramcoMech2.0

C2H6+O2=C2H5+HO2                           4.000E+13   0.000   50900.0

REV /  3.000E+11   0.000       0.0 /

C2H6+HO2=C2H5+H2O2                         1.700E+13   0.000   20460.0

REV /  1.069E+11   0.240    7842.0 /

CH3+C2H3=CH4+C2H2                          3.920E+11   0.000       0.0

REV /  2.962E+13   0.000   66050.0 /

CH3+C2H5=CH4+C2H4                          1.950E+13  -0.500       0.0

REV /  2.895E+16  -0.700   70170.0 /

C2H3+H=C2H2+H2                             1.700E+14   0.000       0.0 ! AramcoMech2.0

CH3+CH3=H+C2H5                             4.740E+12   0.105   10664.0 ! AramcoMech2.0

PLOG / 1.000E-02   4.740E+12   0.105   10664.3 /

PLOG / 1.000E-01   2.570E+13  -0.096   11406.1 /

PLOG / 1.000E+00   3.100E+14  -0.362   13372.5 /

PLOG / 1.000E+01   2.150E+10   0.885   13532.5 /

PLOG / 1.000E+02   1.032E+02   3.230   11236.1 /

C2H3+O2=>CH2O+H+CO                         2.271E+14  -0.7498   3155.

PLOG / 0.1   5.582E+16  -1.5524   1014. / ! Goldsmith2015

PLOG / 1.    1.111E+17  -1.6331   1227. /

PLOG / 10.   1.109E+18  -1.8801   2771. /

PLOG / 100.  2.271E+14  -0.7498   3155. /

C2H3+O2=CH2O+HCO                           9.732E+13  -0.7498   3155.

PLOG /    0.1 2.392E+16  -1.5524   1014.  / ! Goldsmith2015

PLOG /    1.  4.761E+16  -1.6331   1227.  /

PLOG /   10.  4.754E+17  -1.8801   2771.  /

PLOG /  100.  9.732E+13  -0.7498   3155.  /

C2H6=C2H5+H                                2.783E+21  -1.560  103800.0

REV /  3.610E+13   0.000       0.0 /

C2H4+CH3=C2H3+CH4                          6.620E+00   3.700    9500.0

REV /  1.440E+00   4.020    5472.0 /

CH3CO(+M)=CH3+CO(+M)                       3.000E+12   0.000   16720.0

LOW /  1.200E+15   0.000   12518.0 /

CH3CHO=CH3+HCO                             2.614E+15   0.150   80550.0

REV /  2.000E+13   0.000       0.0 /

CH3CHO+O2=CH3CO+HO2                        3.010E+13   0.000   39150.0

REV /  8.552E+10   0.320   -1940.0 /

CH3CHO+OH=CH3CO+H2O                        2.340E+10   0.730    1110.0

CH3CHO+OH=CH2CHO+H2O                       1.600E+13   0.000    2000.0

CH3CHO+H=CH3CO+H2                          1.340E+13   0.000    3300.0

REV /  2.096E+12  -0.010   19690.0 /

CH3CHO+O=CH3CO+OH                          5.940E+12   0.000    1868.0

REV /  4.080E+11  -0.010   16170.0 /

CH3CHO+HO2=CH3CO+H2O2                      3.520E-01   4.075    5580.0 ! TST AM2011

REV /  1.050E-01   3.978    3230.0 /

CH3CHO+CH3=CH3CO+CH4                       2.608E+06   1.780    5911.0

REV /  1.066E+07   1.770   22790.0 /

C2H4+O2=C2H3+HO2                           4.000E+13   0.000   58200.0

REV /  4.939E+13  -0.500    1368.0 /

CH+CH4=C2H4+H                              6.000E+13   0.000       0.0

REV /  3.573E+14   0.000   55480.0 /

CH3CO+H=CH2CO+H2                           2.000E+13   0.000       0.0

REV /  7.270E+09   0.000   83040.0 /

CH3CO+O=CH2CO+OH                           2.000E+13   0.000       0.0

REV /  7.270E+09   0.000   83040.0 /

CH3CO+CH3=CH2CO+CH4                        5.000E+13   0.000       0.0

REV /  7.270E+09   0.000   83040.0 /

C2H4+O=CH2CHO+H                            3.390E+06   1.880     179.0

REV /  9.481E+06   1.790   16050.0 /

C2H5+O=>CH3+CH2O                           5.000E+13   0.000       0.0

C2H5O=CH3+CH2O                             1.320E+20  -2.02    20750.

C2H5O+O2=CH3CHO+HO2                        4.280E+10   0.000    1097.0

REV /  3.872E+08   0.440   31880.0 /

C2H3+O2=CH2CHO+O                           1.022E+11   0.4997   3621.

PLOG /    0.1 1.584E+11   0.3279    360.  / ! Goldsmith2015

PLOG /    1.  5.196E+11   0.1858    809.  /

PLOG /   10.  1.020E+13  -0.1482   2483.  /

PLOG /  100.  1.022E+11   0.4997   3621.  /

C2H5+O2(+M)=C2H5OO(+M)                     6.890E+16  -1.627     200.0

LOW /  3.106E+36  -6.508     595.3 /

TROE / 0.3200 50.0  50000.0 10000. /

C2H5OOH=C2H5O+OH              6.310E+14   0.000   42300.0

REV /  8.314E+06   1.700   -3207.0 /

C2H5OO+CH2O=C2H5OOH+HCO                    1.990E+12   0.000   11670.0

REV /  8.422E+12  -0.510    6605.0 /

C2H4+CH3OO=C2H3+CH3OOH                     1.130E+13   0.000   30430.0

REV /  3.000E+12   0.000   11500.0 /

CH4+C2H5OO=CH3+C2H5OOH                     1.810E+11   0.000   18480.0

REV /  3.673E+11  -0.510   -1731.0 /

C2H5+HO2=C2H5O+OH                          3.200E+13   0.000       0.0

REV /  3.075E+15  -0.320   27490.0 /

CH3OO+C2H5=CH3O+C2H5O                      7.000E+12   0.000   -1000.0

REV /  6.569E+16  -0.900   31260.0 /

C2H5OO+HO2=C2H5OOH+O2                      1.750E+10   0.000   -3275.0

REV /  5.106E+13  -0.830   34480.0 /

C2H6+CH3OO=C2H5+CH3OOH                     1.700E+13   0.000   20460.0

REV /  7.500E+11   0.000    1280.0 /

C2H6+C2H5OO=C2H5+C2H5OOH                   1.700E+13   0.000   20460.0

REV /  7.500E+11   0.000    1280.0 /

CH3C(O)OO=CH3CO+O2                         4.625E+19  -1.900   39560.0

REV /  1.200E+11   0.000   -1100.0 /

CH3CO2+M=CH3+CO2+M                         4.400E+15   0.000   10500.0

REV /  9.648E+12   0.200   21040.0 /

CH3C(O)OOH=CH3CO2+OH                       5.010E+14   0.000   40150.0

REV /  1.738E+08   1.570    1305.0 /

CH3C(O)OO+HO2=CH3C(O)OOH+O2                1.750E+10   0.000   -3275.0

REV /  4.093E+12  -0.330   39040.0 /

C2H5O=CH3CHO+H                             5.430E+15  -0.69    22230.

H2O2+CH3C(O)OO=HO2+CH3C(O)OOH              2.410E+12   0.000    9936.0

REV /  3.984E+11   0.320   11070.0 /

CH4+CH3C(O)OO=CH3+CH3C(O)OOH               1.810E+11   0.000   18480.0

REV /  2.944E+10   0.000    2831.0 /

C2H6+CH3C(O)OO=C2H5+CH3C(O)OOH             1.700E+13   0.000   20460.0

REV /  1.767E+10   0.560    8980.0 /

CH2O+CH3C(O)OO=HCO+CH3C(O)OOH              1.990E+12   0.000   11670.0

REV /  6.752E+11   0.000   11170.0 /

CH3OO+CH3CHO=CH3OOH+CH3CO                  3.010E+12   0.000   11930.0

REV /  2.519E+13  -0.510    8991.0 /

CH3CHO+CH3C(O)OO=CH3CO+CH3C(O)OOH          3.010E+12   0.000   11930.0

REV /  2.000E+12  -0.020   13150.0 /

C2H5OO=C2H4+HO2                            3.370E+55 -13.420   44670.0

REV /  1.392E+49 -12.100   21620.0 /

CH2CHO=CH2CO+H                             3.094E+15  -0.260   50820.0

REV /  5.000E+13   0.000   12300.0 /

CH2CHO+O2=>CH2O+CO+OH                      6.400E+10   0.       9000.0

CH2CHO+O2=>CH2CO+HO2                       1.024E+12   0.       9000.0

HCCO+O2=>OH+CO+CO                          1.910E+11  -0.020    1020.0 ! AramcoMech2.0

HCCO+O2=>CO2+CO+H                          4.780E+12  -0.142    1150.0 ! AramcoMech2.0

CH+CO+M<=>HCCO+M                           7.570E+22  -1.900       0.0 ! AramcoMech2.0

CH+HCCO<=>CO+C2H2                          5.000E+13   0.000       0.0 ! AramcoMech2.0

C2H4+H2=CH3+CH3                            3.767E+12   0.830   84710.0

REV /  1.000E+14   0.000   32000.0 /

pC2H4OH=C2H4+OH                            1.293E+12  -0.370   26850.0

REV /  9.930E+11   0.000    -960.0 /

iC3H7+HO2=>CH3CHO+CH3+OH                   7.000e+12   0.000   -1000.0 !iC3H7O=CH3CHO+CH3

iC3H7+CH3OO=>CH3CHO+CH3+CH3O               7.000e+12   0.000   -1000.0 !iC3H7O=CH3CHO+CH3

C3H6+H=iC3H7                               2.000E+13   0.000    1200.0 !rev

C3H6+H<=>aC3H5+H2                          3.644E+05   2.455    4361.2 !

C3H6+O=>C2H5+HCO                           7.300e+12   0.000    1710.0

C3H6+OH=aC3H5+H2O                          3.120E+06   2.000    -298.0 !

C3H6+HO2=aC3H5+H2O2                        1.500e+11   0.000   14190.0   ! KUCRSwpi

rev /  1.536e+12  -0.362   13842.6 /

C3H6+CH3=aC3H5+CH4                         1.400e+11   0.000    8800.0

C3H6+O2=aC3H5+HO2                          4.000e+13   0.000   50900.0   ! KUCRSwpi

rev /  1.835e+13  -0.217   11454.7 /

C3H6=C2H3+CH3                              5.986E+30  -3.9888 107791.    ! HPL

PLOG /    0.1 3.075E+69 -15.9470 123807.  / ! rrkm2014

PLOG /    1.  1.898E+71 -16.0387 128780.  /

PLOG /   10.  1.863E+69 -15.1393 131253.  /

PLOG /  100.  7.007E+61 -12.8256 128945.  /

C3H6=aC3H5+H                               7.701E+19  -1.2276  90816.    ! HPL

PLOG /    0.1 3.235E+67 -15.3226 115930.  / ! rrkm2014

PLOG /    1.  2.127E+66 -14.7050 118634.  /

PLOG /   10.  2.694E+61 -13.0997 118235.  /

PLOG /  100.  9.554E+51 -10.2956 113532.  /

aC3H5+O2=>CH2O+CH2CHO                      4.570E+12   0.000   18800.0 !

aC3H5+HO2=>CH2O+C2H3+OH                    9.600E+12   0.000    1000.0 !aC3H5O=>CH2O+C2H3

aC3H5+CH3OO=>CH2O+C2H3+CH3O                9.600E+12   0.000    1000.0 !aC3H5O=>CH2O+C2H3

aC3H5+H=C2H3+CH3                           1.994E+37  -6.0039  33164.0

PLOG /    0.1 6.112E+27  -3.5667  18980.  / ! rrkm2014

PLOG /    1.  1.890E+32  -4.7341  24894.  /

PLOG /   10.  1.994E+37  -6.0039  33164.  /

PLOG /  100.  5.632E+37  -6.0098  39080.  /

END

iC4H9=iC4H8+H                              1.000E+14    0.000   37560.6 !

iC4H9+O2=iC4H8+HO2                         7.140E+15   -1.210   21050.0 !

iC4H8+OH=iC4H7+H2O                         1.000E+07    2.000    -298.0 !

iC4H8+HO2=iC4H7+H2O2                       9.000E+08    1.485   15419.4 !

iC4H8=iC4H7+H                              3.000E+32   -4.795   97024.0 !

iC4H7=>C2H4+C2H3                           3.000E+47   -9.740   74360.0 !

iC4H7+O2=>C2H4+CH2O+HCO                    7.140E+15   -1.210   21050.0 !

iC4H7+HO2=>CH3CHO+C2H3+OH                  2.000E+13    0.000    1000.0 !

!

!ethanol mechanism

!

C2H5OH(+M)=C2H4+H2O(+M)                    2.726E+05    2.3169  62954.0 !

LOW/6.090E+90  -21.0692  87325.0/!

TROE / 0.9 50000 3500 /

H2O/5/

C2H5OH(+M)=CH2OH+CH3(+M)                   1.916E+29   -3.7818  92416.0 !

LOW /  6.440E+58 -11.2274  99333.0 /

TROE / 0.7 200 350 3500 /

H2/2/ H2O/5/ CO/2/ CO2/3/

C2H5OH+O2=pC2H4OH+HO2                      8.910E+01    3.124   45860.0 !

C2H5OH+O2=sC2H4OH+HO2                      4.860E+01    3.330   37020.0 !

C2H5OH+O2=C2H5O+HO2                        6.630E+03    2.892   49190.0 !

C2H5OH+OH=pC2H4OH+H2O                      5.130E+01    3.203   -1760.0 !

C2H5OH+OH=sC2H4OH+H2O                      3.555E+04    2.526   -1900.0 !

C2H5OH+OH=C2H5O+H2O                        4.290E+02    2.906     650.0 !

C2H5OH+HO2=sC2H4OH+H2O2                    2.220E-02    4.161    7060.0 !

C2H5OH+HO2=pC2H4OH+H2O2                    7.350E-03    4.021   14280.0 !

C2H5OH+HO2=C2H5O+H2O2                      1.450E-01    3.822   18430.0 !

C2H5OH+H=pC2H4OH+H2                        1.230E+07    1.800    5098.0 !

C2H5OH+H=sC2H4OH+H2                        2.580E+07    1.650    2827.0 !

C2H5OH+H=C2H5O+H2                          1.500E+07    1.600    3038.0 !

C2H5OH+CH3OO=C2H5O+CH3OOH                  1.260E+12    0.000   20440.0 !

C2H5OH+CH3OO=pC2H4OH+CH3OOH                8.400E+12    0.000   20440.0 !

C2H5OH+CH3OO=sC2H4OH+CH3OOH                1.400E+13    0.000   16340.0 !

sC2H4OH+O2=CH3CHO+HO2                      3.810E+06    2.000    1641.0 !

CH2OH+O2=CH2O+HO2                          3.810E+06    2.000    1641.0 !

!

!cross (RH+R'=R+R'OOH, R+R'OO=RO+R'O, ROO+R'OO=RO+R'O+O2, RO and ROOH decomp)

!

!PRF+ethanol

nC7H16+iC8H17OO=nC7H15+iC8H17OOH           9.496E+12    0.177   17684.1 !

iC8H18+nC7H15OO=iC8H17+nC7H15OOH           1.348E+11    0.692   16979.3 !

C2H5OH+nC7H15OO=C2H5O+nC7H15OOH            1.260E+12    0.000   20440.0 !

C2H5OH+nC7H15OO=pC2H4OH+nC7H15OOH          8.400E+12    0.000   20440.0 !

C2H5OH+nC7H15OO=sC2H4OH+nC7H15OOH          1.400E+13    0.000   16340.0 !

C2H5OH+iC8H17OO=C2H5O+iC8H17OOH            1.260E+12    0.000   20440.0 !

C2H5OH+iC8H17OO=pC2H4OH+iC8H17OOH          8.400E+12    0.000   20440.0 !

C2H5OH+iC8H17OO=sC2H4OH+iC8H17OOH          1.400E+13    0.000   16340.0 !

nC7H15+iC8H17OO=>nC7H15O+iC8H17O           7.000E+12    0.000   -1000.0 !

iC8H17+nC7H15OO=>iC8H17O+nC7H15O           7.000E+12    0.000   -1000.0 !

nC7H15OO+iC8H17OO=>nC7H15O+iC8H17O+O2      1.400E+16   -1.610    1860.0 !

nC7H15O=>CH2O+nC5H10+CH3                   1.000E+11    0.000   11900.0 !

nC7H15OOH=nC7H15O+OH                       3.000E+15    0.000   41600.0 !

iC8H17O=>C2H4+CH3CHO+iC4H9                 1.000E+11    0.000   11900.0 !

iC8H17OOH=iC8H17O+OH                       3.000E+15    0.000   41600.0 !

!

!base mechanism

!

O2+H=OH+O                                  9.756E+13    0.000   14844.6

H2+O=OH+H                                  5.120E+04    2.670    6278.7

OH+OH=O+H2O                                1.510E+09    1.140     100.4

H2O+H=H2+OH                                4.520E+08    1.600   18422.6

H2O2+OH=H2O+HO2                            7.830E+12    0.000    1331.3

H+OH+M=H2O+M                               1.659E+23   -2.000       0.0

Appendix-C.1 Mechanism File

2.38759306E-08-3.57718690E-12-1.35517265E+04 4.08164660E+01                   4

iC7H13            thgen C   7H  13          G   300.000  5000.000 1396.00      1!aC7H13b

2.26058410E+01 2.77383394E-02-9.53221699E-06 1.48480972E-09-8.63512957E-14    2

-4.56407218E+03-9.91382375E+01-3.87510891E+00 9.11860004E-02-6.79894558E-05    3

2.61194736E-08-4.08578594E-12 4.44229599E+03 4.24657120E+01                   4

iC4H9             thgen C   4H   9          G   300.000  5000.000 1371.00      1 !tC4H9

1.06256766E+01 2.10524192E-02-7.24699414E-06 1.12975765E-09-6.57275211E-14    2

8.98003085E+01-3.28154286E+01 3.00651244E+00 2.87404049E-02-2.86301425E-06    3

-5.92506428E-09 1.92085423E-12 3.85126136E+03 1.18650543E+01                   4

iC4H8             thgen C   4H   8          G   300.000  5000.000 1388.00      1 !iC4H8

1.12250420E+01 1.81815694E-02-6.20445208E-06 9.61623502E-10-5.57203448E-14    2

-7.69990589E+03-3.73290465E+01 9.39096232E-01 3.90507649E-02-2.16375684E-05    3

5.86955550E-09-6.13931254E-13-3.74834503E+03 1.91392713E+01                   4

iC4H7             thgen C   4H   7          G   300.000  5000.000 1386.00      1 !iC4H7a

1.20755413E+01 1.53838035E-02-5.34471115E-06 8.38457076E-10-4.89970916E-14    2

1.02789716E+04-4.16938641E+01 2.33005612E-01 4.23571985E-02-2.93968519E-05    3

1.09284827E-08-1.73005596E-12 1.45421013E+04 2.22900171E+01                   4

iC8H17O           thgen C   8H  17O   1     G   300.000  5000.000 1400.00      1 !iC8H17Ob

2.92000599E+01 3.62425322E-02-1.23912392E-05 1.92327737E-09-1.11564645E-13    2

-3.73134757E+04-1.30872815E+02-3.96943499E+00 1.18162148E-01-9.09286549E-05    3

3.65727157E-08-6.01087592E-12-2.62535880E+04 4.56667705E+01                   4

iC8H17OOH         thgen C   8H  18O   2     G   300.000  5000.000 1395.00      1 !iC8H17OOHb

3.33786590E+01 3.76091799E-02-1.30114163E-05 2.03563516E-09-1.18738112E-13    2

-5.71706588E+04-1.51690420E+02-3.94285031E+00 1.31014400E-01-1.04685507E-04    3

4.37791373E-08-7.48600810E-12-4.47722798E+04 4.66425335E+01                   4

!

!ethanol mechanism

!

C2H5OH     3/12/95 THERMC   2H   6O   1     G   300.000  5000.000 1391.00      1

7.95264841E+00 1.31574144E-02-4.43693359E-06 6.82201160E-10-3.93096335E-14    2

-3.23602958E+04-1.76537972E+01 4.23149045E-01 2.92858167E-02-1.73845099E-05    3

5.33639892E-09-6.74917252E-13-2.95683054E+04 2.33538763E+01                   4

sC2H4OH    3/12/95 THERMC   2H   5O   1     G  0300.00   5000.00  1553.00      1

7.26631179E+00 1.09588926E-02-3.63662803E-06 5.53659830E-10-3.17012322E-14    2

-8.64310684E+03-1.06794104E+01 1.83974631E+00 1.87789371E-02-4.60544253E-06    3

-2.13116990E-09 9.43772653E-13-6.29595195E+03 2.01446141E+01                   4

CH2OH                   H   3O   1C   1     G    200.00   6000.00 1000.00      1

4.72398695E+00 6.50832540E-03-2.24240605E-06 3.51742805E-10-2.06424726E-14    2

-3.81156740E+03 1.47350104E+00 4.73864580E+00 1.27132491E-03 1.46005656E-05    3

-1.83666166E-08 7.07708694E-12-3.56200591E+03 2.65277676E+00                   4

!

!base mechanism

!

N2                      N   2               G    200.00   6000.00 1000.00      1

2.95254070E+00 1.39688380E-03-4.92625770E-07 7.86000910E-11-4.60749780E-15    2

-9.23937530E+02 5.87182210E+00 3.53096280E+00-1.23659500E-04-5.02993390E-07    3

2.43527680E-09-1.40879540E-12-1.04696370E+03 2.96743910E+00                   4

O                       O   1               G    200.00   6000.00 1000.00      1

2.54363697E+00-2.73162486E-05-4.19029520E-09 4.95481845E-12-4.79553694E-16    2

2.92260120E+04 4.92229457E+00 3.16826710E+00-3.27931884E-03 6.64306396E-06    3

-6.12806624E-09 2.11265971E-12 2.91222592E+04 2.05193346E+00                   4

O2                      O   2               G    200.00   6000.00 1000.00      1

3.66096083E+00 6.56365523E-04-1.41149485E-07 2.05797658E-11-1.29913248E-15    2

-1.21597725E+03 3.41536184E+00 3.78245636E+00-2.99673415E-03 9.84730200E-06    3

-9.68129508E-09 3.24372836E-12-1.06394356E+03 3.65767573E+00                   4

H                       H   1               G    200.00   6000.00 1000.00      1

2.50000000E+00 0.00000000E+00 0.00000000E+00 0.00000000E+00 0.00000000E+00    2

2.54736600E+04-4.46682850E-01 2.50000000E+00 0.00000000E+00 0.00000000E+00    3

0.00000000E+00 0.00000000E+00 2.54736600E+04-4.46682850E-01                   4

OH                      H   1O   1          G    200.00   6000.00 1000.00      1

2.83864607E+00 1.10725586E-03-2.93914978E-07 4.20524247E-11-2.42169092E-15    2

3.68599690E+03 5.84452662E+00 3.99201543E+00-2.40131752E-03 4.61793841E-06    3

-3.88113333E-09 1.36411470E-12 3.35711894E+03-1.03925458E-01                   4

HO2                     H   1O   2          G    200.00   6000.00 1000.00      1

4.17226590E+00 1.88120980E-03-3.46292970E-07 1.94685160E-11 1.76091530E-16    2

2.13222508E+02 2.95779740E+00 4.30178800E+00-4.74902010E-03 2.11579530E-05    3

-2.42759610E-08 9.29206700E-12 4.46212417E+02 3.71670100E+00                   4

H2                      H   2               G    200.00   6000.00 1000.00      1

2.93283050E+00 8.26598020E-04-1.46400570E-07 1.54098510E-11-6.88796150E-16    2

-8.13055820E+02-1.02431640E+00 2.34430290E+00 7.98042480E-03-1.94779170E-05    3

2.01569670E-08-7.37602890E-12-9.17924130E+02 6.83002180E-01                   4

H2O                     H   2O   1          G    200.00   6000.00 1000.00      1

2.67703890E+00 2.97318160E-03-7.73768890E-07 9.44335140E-11-4.26899910E-15    2

-2.98858940E+04 6.88255000E+00 4.19863520E+00-2.03640170E-03 6.52034160E-06    3

-5.48792690E-09 1.77196800E-12-3.02937260E+04-8.49009010E-01                   4

H2O2                    H   2O   2          G    200.00   6000.00 1000.00      1

4.57333537E+00 4.04984070E-03-1.29479479E-06 1.97281710E-10-1.13402846E-14    2

-1.80040975E+04 7.04278488E-01 4.27611269E+00-5.42822417E-04 1.67335701E-05    3

-2.15770813E-08 8.62454363E-12-1.77035843E+04 3.43505074E+00                   4

CO                      O   1C   1          G    200.00   6000.00 1000.00      1

3.04848590E+00 1.35172810E-03-4.85794050E-07 7.88536440E-11-4.69807460E-15    2

-1.42661170E+04 6.01709770E+00 3.57953350E+00-6.10353690E-04 1.01681430E-06    3

9.07005860E-10-9.04424490E-13-1.43440860E+04 3.50840930E+00                   4

CO2                     O   2C   1          G    200.00   6000.00 1000.00      1

4.63651110E+00 2.74145690E-03-9.95897590E-07 1.60386660E-10-9.16198570E-15    2

-4.90249040E+04-1.93489550E+00 2.35681300E+00 8.98412990E-03-7.12206320E-06    3

2.45730080E-09-1.42885480E-13-4.83719710E+04 9.90090350E+00                   4

CH                      H   1C   1          G    200.00   6000.00 1000.00      1

2.52090620E+00 1.76537260E-03-4.61475810E-07 5.92885670E-11-3.34732090E-15    2

7.10230206E+04 7.40532230E+00 3.48981660E+00 3.23835540E-04-1.68899060E-06    3

3.16217330E-09-1.40609070E-12 7.06888776E+04 2.08401110E+00                   4

HCO                     H   1O   1C   1     G    200.00   6000.00 1000.00      1

3.64896209E+00 3.08090819E-03-1.12429876E-06 1.86308085E-10-1.13951828E-14    2

3.71209048E+03 5.06147406E+00 4.22118584E+00-3.24392532E-03 1.37799446E-05    3

!

! ===== gasoline PRF reduced mechanism 2.0 (#190128a) =====

!       with ethanol

!

! Fuel: nC7H16(n-heptane)

!       iC8H18(isooctane, 2,2,4-trimethylpentane)

!       C2H5OH(ethanol)

!

THERMO

300.000  1000.000  5000.000

!

!n-heptane mechanism

!

nC7H16            thgen C   7H  16          G   300.000  5000.000 1391.00      1 !nC7H16

2.22177923E+01 3.47632604E-02-1.18388528E-05 1.83265875E-09-1.06109932E-13    2

-3.42772591E+04-9.23217457E+01-1.26871064E+00 8.54348955E-02-5.25290944E-05    3

1.62884286E-08-2.02214778E-12-2.56586562E+04 3.53739478E+01                   4

nC7H15            thgen C   7H  15          G   300.000  5000.000 1382.00      1 !nC7H15c

2.16358383E+01 3.23340841E-02-1.09280566E-05 1.68368616E-09-9.71844623E-14    2

-1.05866176E+04-8.52122680E+01-3.88848544E-02 7.56801544E-02-4.07609055E-05    3

9.33601461E-09-4.94482163E-13-2.35589863E+03 3.37383486E+01                   4

nC7H15OO          thgen C   7H  15O   2     G   300.000  5000.000 1390.00      1 !nC7H15OOc

2.60523050E+01 3.43839001E-02-1.18715005E-05 1.85452176E-09-1.08053193E-13    2

-2.94761134E+04-1.05463689E+02 3.59006547E-01 9.42245153E-02-6.66804487E-05    3

2.54724302E-08-4.13086673E-12-2.03728288E+04 3.28613586E+01                   4

nC7H14OOH         thgen C   7H  15O   2     G   300.000  5000.000 1389.00      1 !C7H14OOHk

2.74573337E+01 3.26265036E-02-1.12386517E-05 1.75329980E-09-1.02071273E-13    2

-2.48239746E+04-1.11231599E+02 8.68010527E-01 9.24071780E-02-6.23385915E-05    3

2.16105189E-08-3.07642818E-12-1.53160279E+04 3.24663802E+01                   4

nHOOC7H14OO       thgen C   7H  15O   4     G   300.000  5000.000 1390.00      1 !HOOC7H14OOk

3.28360162E+01 3.32763432E-02-1.15831469E-05 1.81955309E-09-1.06432285E-13    2

-4.40287923E+04-1.36795390E+02 1.41185977E+00 1.10703712E-01-8.70872996E-05    3

3.63452156E-08-6.27852573E-12-3.33720367E+04 3.07923596E+01                   4

nC7KET            thgen C   7H  14O   3     G   300.000  5000.000 1386.00      1 !COC6H13OOHg

2.86566981E+01 3.21565793E-02-1.11959188E-05 1.75889497E-09-1.02888014E-13    2

-6.19744506E+04-1.16883589E+02 2.32186256E+00 9.45240193E-02-6.99211946E-05    3

2.79874447E-08-4.74465002E-12-5.27085951E+04 2.45959611E+01                   4

nC5H9O            thgen C   5H   9O   1     G   300.000  5000.000 1379.00      1 !C4H9COa

1.64012943E+01 2.04003752E-02-6.93873682E-06 1.07367226E-09-6.21614279E-14    2

-1.67278682E+04-5.71188765E+01 2.36413323E+00 4.95786327E-02-2.90721987E-05    3

8.29294303E-09-9.03229011E-13-1.14543903E+04 1.96153510E+01                   4

nC5H11            thgen C   5H  11          G   300.000  5000.000 1390.00      1 !C5H11a

1.52896647E+01 2.39755267E-02-8.18341222E-06 1.26861432E-09-7.35224954E-14    2

-9.72919304E+02-5.44277267E+01 5.79609854E-02 5.60823625E-02-3.31785211E-05    3

9.79021087E-09-1.14223950E-12 4.71440753E+03 2.86918308E+01                   4

nC5H10            thgen C   5H  10          G   300.000  5000.000 1392.00      1 !C5H10a

1.45837757E+01 2.24057409E-02-7.63231344E-06 1.18163580E-09-6.84206955E-14    2

-1.00871422E+04-5.23577225E+01-1.07237458E+00 5.74872482E-02-3.75589723E-05    3

1.28055098E-08-1.81083209E-12-4.46482377E+03 3.23144603E+01                   4

nC5H9             thgen C   5H   9          G   300.000  5000.000 1388.00      1 !aC5H9a

1.45041351E+01 2.05083694E-02-7.10654706E-06 1.11295524E-09-6.49627735E-14    2

6.79087910E+03-5.34346369E+01-1.04797696E+00 5.41768083E-02-3.45284353E-05    3

1.11684416E-08-1.47643393E-12 1.25116524E+04 3.11242264E+01                   4

nC7H15O           thgen C   7H  15O   1     G   300.000  5000.000 1395.00      1 !nC7H15Oc

2.43050592E+01 3.31299633E-02-1.13048045E-05 1.75223806E-09-1.01542653E-13    2

-2.88617678E+04-9.94127348E+01-1.08852878E+00 9.23114953E-02-6.44357297E-05    3

2.36757686E-08-3.61001227E-12-1.99930305E+04 3.70751744E+01                   4

nC7H15OOH         thgen C   7H  16O   2     G   300.000  5000.000 1390.00      1 !nC7H15OOHc

2.85696040E+01 3.45362690E-02-1.19635215E-05 1.87319578E-09-1.09321273E-13    2

-4.87912635E+04-1.20812558E+02-5.93636262E-01 1.02847108E-01-7.42877994E-05    3

2.83328117E-08-4.51137547E-12-3.85781122E+04 3.59212486E+01                   4

!

!isooctane mechanism

!

iC8H18            thgen C   8H  18          G   300.000  5000.000 1397.00      1 !iC8H18

2.70920030E+01 3.79052142E-02-1.29375691E-05 2.00580387E-09-1.16261109E-13    2

-4.27201269E+04-1.22972430E+02-4.20184373E+00 1.11525135E-01-7.94018605E-05    3

2.94327964E-08-4.47928170E-12-3.19113977E+04 4.48981238E+01                   4

iC8H17            thgen C   8H  17          G   300.000  5000.000 1393.00      1 !iC8H17b

2.64552622E+01 3.55440069E-02-1.20528460E-05 1.86101603E-09-1.07579175E-13    2

-1.90011197E+04-1.16235434E+02-3.08982602E+00 1.02313728E-01-6.84786443E-05    3

2.30138816E-08-3.06907219E-12-8.59119871E+03 4.31119876E+01                   4

iC8H17OO          thgen C   8H  17O   2     G   300.000  5000.000 1394.00      1 !iC8H17OOb

3.09355421E+01 3.74105002E-02-1.29073110E-05 2.01549577E-09-1.17402236E-13    2

-3.78977875E+04-1.36798535E+02-3.06443818E+00 1.22613369E-01-9.71774181E-05    3

4.08857690E-08-7.08508839E-12-2.65537797E+04 4.39441843E+01                   4

iC8H16OOH         thgen C   8H  17O   2     G   300.000  5000.000 1395.00      1 !C8H16OOHg

3.29642834E+01 3.54528218E-02-1.22763166E-05 1.92179615E-09-1.12146066E-13    2

-3.21060047E+04-1.46610031E+02-3.22291546E+00 1.26638844E-01-1.02432626E-04    3

4.32601240E-08-7.45097857E-12-2.01505706E+04 4.54684584E+01                   4

iHOOC8H16OO       thgen C   8H  17O   4     G   300.000  5000.000 1393.00      1 !HOOC8H16OOg

3.68921902E+01 3.70313467E-02-1.28759800E-05 2.02124832E-09-1.18176812E-13    2

-5.00657935E+04-1.62652799E+02-1.32886825E+00 1.32867168E-01-1.07401821E-04    3

4.53903634E-08-7.84223845E-12-3.73590684E+04 4.04443512E+01                   4

iC8KET            thgen C   8H  16O   3     G   300.000  5000.000 1393.00      1 !COC7H15OOHc

3.37013249E+01 3.51404946E-02-1.22403555E-05 1.92384697E-09-1.12581529E-13    2

-6.78193788E+04-1.48813885E+02-3.69054288E+00 1.27292525E-01-1.00891530E-04    3

4.14152491E-08-6.93785042E-12-5.52794020E+04 5.03601636E+01                   4

iC7H13O           thgen C   7H  13O   1     G   300.000  5000.000 1394.00      1 !C6H13vCOb

2.42679982E+01 2.86333840E-02-9.88040093E-06 1.54317611E-09-8.99095196E-14    2

-3.05478683E+04-1.04458208E+02-1.80946874E+00 9.01521048E-02-6.57659227E-05    3

2.48425015E-08-3.84914862E-12-2.15441753E+04 3.53937659E+01                   4

iC7H14            thgen C   7H  14          G   300.000  5000.000 1395.00      1 !C7H14b

2.24912357E+01 2.96685986E-02-1.00455975E-05 1.54950613E-09-8.95071232E-14    2

-2.24019669E+04-9.74583681E+01-3.30938880E+00 9.05113458E-02-6.47606480E-05    3

4.65870470E+00 4.88409490E-03-1.60835630E-06 2.46987870E-10-1.38615050E-14    2

2.56632180E+04-3.99790740E+00 8.08691080E-01 2.33613950E-02-3.55166360E-05    3

2.80145660E-08-8.50044590E-12 2.63327640E+04 1.39396710E+01                   4

CH2CO                   H   2O   1C   2     G    200.00   6000.00 1000.00      1

5.75779010E+00 6.34965070E-03-2.25844070E-06 3.62084620E-10-2.15690300E-14    2

-8.20537635E+03-6.10640370E+00 2.14011650E+00 1.80883680E-02-1.73242160E-05    3

9.27674770E-09-1.99150110E-12-7.26981595E+03 1.21986990E+01                   4

C2H3                    H   3C   2          G    200.00   6000.00 1000.00      1

4.70253100E+00 7.26422830E-03-2.58019920E-06 4.13199440E-10-2.45914920E-14    2

3.40296750E+04-1.42937140E+00 3.00196020E+00 3.03043540E-03 2.44443150E-05    3

-3.58102420E-08 1.51087000E-11 3.48681730E+04 9.33044950E+00                   4

CH2CHO                  H   3O   1C   2     G    300.00   5000.00 1000.00      1

5.97566990E+00 8.13059140E-03-2.74362450E-06 4.07030410E-10-2.17601710E-14    2

-1.26585075E+03-5.03208790E+00 3.40906240E+00 1.07385740E-02 1.89149250E-06    3

-7.15858310E-09 2.86738510E-12-2.34695926E+02 9.57145350E+00                   4

CH3CO             120186C   2H   3O   1     G  0300.00   5000.00  1000.00      1

0.05612279E+02 0.08449886E-01-0.02854147E-04 0.04238376E-08-0.02268404E-12    2

-0.05187863E+05-0.03274949E+02 0.03125278E+02 0.09778220E-01 0.04521448E-04    3

-0.09009462E-07 0.03193718E-10-0.04108508E+05 0.01122885E+03                   4

CH3CO2     2/14/95 THERMC   2H   3O   2     G   300.000  5000.000 1395.00      1

8.54059736E+00 8.32951214E-03-2.84722010E-06 4.41927196E-10-2.56373394E-14    2

-2.97290678E+04-2.03883545E+01 1.37440768E+00 2.49115604E-02-1.74308894E-05    3

6.24799508E-09-9.09516835E-13-2.72330150E+04 1.81405454E+01                   4

CH3C(O)OO  6/26/95 THERMC   2H   3O   3     G   300.000  5000.000 1396.00      1

1.15641474E+01 8.02672958E-03-2.77497463E-06 4.34069277E-10-2.53202435E-14    2

-1.93142188E+04-3.13182392E+01 2.61619505E+00 2.94830898E-02-2.23952554E-05    3

8.56474251E-09-1.31255496E-12-1.62923875E+04 1.64911398E+01                   4

C2H4                    H   4C   2          G    300.00   5000.00 1000.00      1

4.39854530E+00 9.62286070E-03-3.16637760E-06 4.57476280E-10-2.36594060E-14    2

4.11532030E+03-2.46274380E+00 1.21766000E+00 1.30026750E-02 3.50374470E-06    3

-1.11555140E-08 4.72032220E-12 5.33738280E+03 1.54801690E+01                   4

CH3CHO            120186C   2O   1H   4     G  0300.00   5000.00  1000.00      1

0.05868650E+02 0.01079424E+00-0.03645530E-04 0.05412912E-08-0.02896844E-12    2

-0.02264569E+06-0.06012946E+02 0.02505695E+02 0.01336991E+00 0.04671953E-04    3

-0.01128140E-06 0.04263566E-10-0.02124589E+06 0.01335089E+03                   4

CH3C(O)OOH 6/26/95 THERMC   2H   4O   3     G   300.000  5000.000 1391.00      1

1.25060485E+01 9.47789695E-03-3.30402246E-06 5.19630793E-10-3.04233568E-14    2

-4.59856703E+04-3.79195947E+01 2.24135876E+00 3.37963514E-02-2.53887482E-05    3

9.67583587E-09-1.49266157E-12-4.24677831E+04 1.70668133E+01                   4

C2H5                    H   5C   2          G    200.00   6000.00 1000.00      1

4.28788140E+00 1.24338930E-02-4.41391190E-06 7.06541020E-10-4.20351360E-14    2

1.23269670E+04 8.46025830E-01 4.30585800E+00-4.18336380E-03 4.97072700E-05    3

-5.99058740E-08 2.30484780E-11 1.31122260E+04 4.71002360E+00                   4

pC2H4OH    3/12/95 THERMC   2H   5O   1     G  0300.00   5000.00  1391.00      1

7.52241939E+00 1.10492715E-02-3.72576465E-06 5.72827397E-10-3.30061759E-14    2

-7.29333590E+03-1.24958732E+01 1.17714711E+00 2.48115685E-02-1.50299503E-05    3

4.79006785E-09-6.40994211E-13-4.95369043E+03 2.20081586E+01                   4

C2H5O      3/12/95 THERMC   2H   5O   1     G   300.000  5000.000 1389.00      1

7.87339772E+00 1.13072907E-02-3.84421421E-06 5.94414105E-10-3.43894538E-14    2

-6.07274953E+03-1.73416790E+01 4.94420708E-01 2.71774434E-02-1.65909010E-05    3

5.15204200E-09-6.48496915E-13-3.35252925E+03 2.28079378E+01                   4

C2H5OO     7/19/ 0 THERMC   2H   5O   2     G   300.000  5000.000 1382.00      1

9.48696023E+00 1.24472545E-02-4.32161576E-06 6.77583033E-10-3.95784568E-14    2

-7.82481795E+03-2.30613020E+01 2.26846188E+00 2.76942578E-02-1.70804106E-05    3

5.87951878E-09-9.20899069E-13-5.03880758E+03 1.64773972E+01                   4

C2H6                    H   6C   2          G    300.00   5000.00 1000.00      1

4.70288470E+00 1.40426350E-02-4.64693770E-06 6.74737380E-10-3.50893120E-14    2

-1.26719880E+04-4.54339500E+00 1.53952600E+00 1.50408410E-02 6.68471150E-06    3

-1.33829480E-08 4.85613980E-12-1.12487660E+04 1.41073750E+01                   4

C2H5OOH    7/19/ 0 THERMC   2H   6O   2     G   300.000  5000.000 1383.00      1

1.19442992E+01 1.26271597E-02-4.41776755E-06 6.96327353E-10-4.08268961E-14    2

-2.57408634E+04-3.73382394E+01 1.29970294E+00 3.65488420E-02-2.52808875E-05    3

9.19746803E-09-1.41362233E-12-2.18866647E+04 2.02719078E+01                   4

aC3H5             thgen C   3H   5          G   300.000  5000.000 1386.00      1

8.86716623E+00 1.09017161E-02-3.80111689E-06 5.97754270E-10-3.49908641E-14    2

1.61010768E+04-2.50805399E+01-3.38203444E-01 3.23181416E-02-2.33791351E-05    3

9.01780357E-09-1.46769445E-12 1.93623652E+04 2.44831718E+01                   4

C3H6              thgen C   3H   6          G   300.000  5000.000 1388.00      1

8.01816543E+00 1.36989847E-02-4.66102270E-06 7.20993895E-10-4.17206749E-14    2

-1.87849595E+03-2.00297954E+01 3.92921186E-01 2.89160669E-02-1.54920156E-05    3

3.88742327E-09-3.37241003E-13 1.06771101E+03 2.19072716E+01                   4

iC3H7             thgen C   3H   7          G   300.000  5000.000 1999.00      1

7.09001153E+00 1.70236198E-02-5.94979925E-06 9.40957054E-10-5.53932270E-14    2

6.75927916E+03-1.26897658E+01 6.35981674E-01 2.84995364E-02-1.14520937E-05    3

8.92803184E-10 3.13189892E-13 9.22197497E+03 2.30735195E+01                   4

END

Appendix-C.2 Thermodynamics File

-1.33144093E-08 4.33768865E-12 3.83956496E+03 3.39437243E+00                   4

CH2                     H   2C   1          G    200.00   6000.00 1000.00      1

2.53871220E+00 3.82254910E-03-1.28613040E-06 1.98003080E-10-1.14657430E-14    2

4.61292530E+04 8.10546480E+00 4.17936550E+00-2.21785530E-03 7.96536020E-06    3

-6.91273390E-09 2.24753180E-12 4.57508570E+04-7.61137030E-03                   4

CH2O                    H   2O   1C   1     G    200.00   6000.00 1000.00      1

3.16948070E+00 6.19327420E-03-2.25059810E-06 3.65982450E-10-2.20154100E-14    2

-1.44784250E+04 6.04235330E+00 4.79370360E+00-9.90815180E-03 3.73214590E-05    3

-3.79279020E-08 1.31770150E-11-1.43089550E+04 6.02887020E-01                   4

CH3                     H   3C   1          G    200.00   6000.00 1000.00      1

2.96697350E+00 5.79366720E-03-1.96948090E-06 3.05569360E-10-1.77678430E-14    2

1.65131329E+04 4.79188030E+00 3.67333750E+00 2.00205590E-03 5.78531350E-06    3

-6.98730540E-09 2.60555990E-12 1.64161969E+04 1.60183150E+00                   4

CH3O                    H   3O   1C   1     G    200.00   6000.00 1000.00      1

4.26533080E+00 7.85764060E-03-2.84104380E-06 4.60451900E-10-2.76319060E-14    2

1.65605687E+02 3.93099470E-01 3.26523370E+00 3.30316650E-03 1.70488010E-05    3

-2.27096300E-08 8.80717680E-12 8.38550421E+02 7.42573570E+00                   4

CH3OO      7/13/98 THERMC   1H   3O   2     G   300.000  5000.000 1385.00      1

5.95787891E+00 7.90728626E-03-2.68246234E-06 4.13891337E-10-2.39007330E-14    2

-1.53574838E+03-4.71963886E+00 4.26146906E+00 1.00873599E-02-3.21506184E-06    3

2.09409267E-10 4.18339103E-14-6.84394259E+02 5.16330320E+00                   4

CH4                     H   4C   1          G    200.00   6000.00 1000.00      1

1.63542560E+00 1.00844310E-02-3.36923690E-06 5.34972800E-10-3.15528170E-14    2

-1.00056030E+04 9.99369530E+00 5.14987920E+00-1.36710080E-02 4.91801300E-05    3

-4.84744030E-08 1.66694410E-11-1.02466480E+04-4.64132440E+00                   4

CH3OOH     7/13/98 THERMC   1H   4O   2     G   300.000  5000.000 1390.00      1

8.43117091E+00 8.06817909E-03-2.77094921E-06 4.31332243E-10-2.50692146E-14    2

-1.96678771E+04-1.91170629E+01 3.23442817E+00 1.90129767E-02-1.13386287E-05    3

3.40306653E-09-4.11830222E-13-1.77197926E+04 9.25623949E+00                   4

HCCO                    H   1O   1C   2     G    200.00   6000.00 1000.00      1

5.84690060E+00 3.64059600E-03-1.29590070E-06 2.07969190E-10-1.24000220E-14    2

1.92484960E+04-5.29165330E+00 2.33501180E+00 1.70100830E-02-2.20188670E-05    3

1.54064470E-08-4.34550970E-12 2.00502990E+04 1.19767290E+01                   4

C2H2                    H   2C   2          G    200.00   6000.00 1000.00      1

Appendix-C.3 Transport File

!

! ===== gasoline PRF reduced mechanism 2.0 (#190128a) =====

!       with ethanol

!

! Fuel: nC7H16(n-heptane)

!       iC8H18(isooctane, 2,2,4-trimethylpentane)

!       C2H5OH(ethanol)

!

!n-heptane mechanism

!

nC7H16             2   415.4     6.094     0.0       0.0      1.0 !nC7H16

nC7H15             2   415.4     6.094     0.0       0.0      1.0 !nC7H15c

nC7H15OO           2   506.2     6.238     0.0       0.0      1.0 !nC7H15OOc

nC7H14OOH          2   506.2     6.238     0.0       0.0      1.0 !C7H14OOHk

nHOOC7H14OO        2   596.0     6.376     0.0       0.0      1.0 !HOOC7H14OOk

nC7KET             2   556.3     6.265     0.0       0.0      1.0 !COC6H13OOHg

nC5H9O             2   425.1     5.605     0.0       0.0      1.0 !C4H9COa

nC5H11             2   378.3     5.507     0.0       0.0      1.0 !C5H11a

nC5H10             2   378.3     5.395     0.0       0.0      1.0 !C5H10a

nC5H9              2   378.3     5.395     0.0       0.0      1.0 !aC5H9a

nC7H15O            2   488.4     6.156     0.0       0.0      1.0 !nC7H15Oc

nC7H15OOH          2   506.2     6.238     0.0       0.0      1.0 !nC7H15OOHc

!

!isooctane mechanism

!

iC8H18             2   442.4     6.274     0.0       0.0      1.0 !iC8H18

iC8H17             2   442.4     6.274     0.0       0.0      1.0 !iC8H17b

iC8H17OO           2   530.7     6.410     0.0       0.0      1.0 !iC8H17OOb

iC8H16OOH          2   530.7     6.410     0.0       0.0      1.0 !C8H16OOHg

iHOOC8H16OO        2   616.7     6.565     0.0       0.0      1.0 !HOOC8H16OOg

iC8KET             2   578.1     6.461     0.0       0.0      1.0 !COC7H15OOHc

iC7H13O            2   475.3     6.070     0.0       0.0      1.0 !C6H13vCOb

iC7H14             2   432.5     5.943     0.0       0.0      1.0 !C7H14b

iC7H13             2   421.9     5.948     0.0       0.0      1.0 !aC7H13b

iC4H9              2   362.0     5.120     0.0       0.0      1.0 !tC4H9

iC4H8              2   361.9     5.038     0.0       0.0      1.0 !iC4H8

iC4H7              2   361.9     5.038     0.0       0.0      1.0 !iC4H7a

iC8H17O            2   513.4     6.332     0.0       0.0      1.0 !iC8H17Ob

iC8H17OOH          2   530.7     6.410     0.0       0.0      1.0 !iC8H17OOHb

!

!ethanol mechanism

!

C2H5OH             2   470.6     4.410     0.000     0.00     1.5 ! NMM

sC2H4OH            2   470.6     4.410     0.000     0.00     1.5 ! NMM

CH2OH              2   417.0     3.690     1.700     0.00     2.0

!

!base mehcanism

!

N2                 1    97.53    3.621     0.000     1.76     4.0

O                  0    80.0     2.750     0.000     0.00     0.0

O2                 1   107.4     3.458     0.000     1.60     3.8

H                  0   145.0     2.050     0.000     0.00     0.0

OH                 1    80.0     2.750     0.000     0.00     0.0

HO2                2   107.4     3.458     0.000     0.00     1.0

H2                 1    38.0     2.920     0.000     0.79   280.0

H2O                2   572.4     2.605     1.844     0.00     4.0

H2O2               2   107.4     3.458     0.000     0.00     3.8

CO                 1    98.1     3.650     0.000     1.95     1.8

CO2                1   244.0     3.763     0.000     2.65     2.1

CH                 1    80.0     2.750     0.000     0.00     0.0

HCO                2   498.0     3.590     0.000     0.00     0.0

CH2                1   144.0     3.800     0.000     0.00     0.0

CH2O               2   498.0     3.590     0.000     0.00     2.0

CH3                1   144.0     3.800     0.000     0.00     0.0

CH3O               2   417.0     3.690     1.700     0.00     2.0

CH3OO              2   481.8     3.626     0.000     0.00     1.0 ! WJP

CH4                2   141.4     3.746     0.000     2.60    13.0

CH3OOH             2   481.8     3.626     0.000     0.00     1.0 ! WJP

HCCO               2   150.0     2.500     0.000     0.00     1.0 ! *

C2H2               1   265.3     3.721     0.000     0.00     2.5 ! NMM

CH2CO              2   314.3     4.191     1.4       0.0      2.0 ! WJP

C2H3               2   265.3     3.721     0.000     0.00     1.0 ! NMM

CH2CHO             2   436.0     3.970     0.000     0.00     2.0

CH3CO              2   436.0     3.970     0.000     0.00     2.0

CH3CO2             2   436.0     3.970     0.000     0.00     2.0 ! WJP

CH3C(O)OO          2   436.0     3.970     0.000     0.00     2.0 ! WJP

C2H4               2   238.4     3.496     0.000     0.00     1.5 ! NMM

CH3CHO             2   436.0     3.970     0.000     0.00     2.0

CH3C(O)OOH         2   436.0     3.970     0.000     0.00     2.0 ! WJP

C2H5               2   247.5     4.350     0.000     0.00     1.5 ! NMM

pC2H4OH            2   470.6     4.410     0.000     0.00     1.5 ! NMM

C2H5O              2   470.6     4.410     0.000     0.00     1.5 ! NMM

C2H5OO             2   470.6     4.410     0.000     0.00     1.5 ! WJP

C2H6               2   247.5     4.350     0.000     0.00     1.5 ! NMM

C2H5OOH            2   470.6     4.410     0.000     0.00     1.5 ! WJP

aC3H5              2   339.3     4.606     0.0       0.0      1.0

C3H6               2   339.3     4.606     0.0       0.0      1.0

iC3H7              2   339.3     4.758     0.0       0.0      1.0

END

Appendix-D

Laminar Flame Speed using Cantera

import cantera as ct

# Inlet Temperature in Kelvin and Inlet Pressure in Pascals

To = 1150

Po = 3*101325

# Define the gas-mixture and kinetics

gas = ct.Solution("nheptane_knock.yaml")

gas.set_equivalence_ratio(1.0, "NC7H16", {"O2": 1.0, "N2": 3.76})

gas. TP = To, Po

# Domain width in metres

width = 0.014

# Create the flame object

flame = ct.FreeFlame(gas, width=width)

# Define tolerances for the solver

flame.set_refine_criteria(ratio=3, slope=0.1, curve=0.1)

# Define logging level

loglevel = 1

flame.solve(loglevel=loglevel, auto=True)

Su0 = flame.velocity[0]

print(f"Flame Speed is: {Su0 * 100:.2f} cm/s")

Output

Flame Speed is: 2955.96 cm/s

Appendix-E

C-J Detonation Speed using Cantera

import cantera as ct

from sdtoolbox. post-shock import CJspeed

from sdtoolbox. post-shock import PostShock_eq

from sdtoolbox.thermo import soundspeed_eq, soundspeed_fr

# Initial state specification:

# P1 = Initial Pressure

# T1 = Initial Temperature

# U = Shock Speed

# q = Initial Composition

# mech = Cantera mechanism File name

# stoichiometric H2-air detonation at nominal atmospheric conditions

P1 = 3*101325.

T1 = 1150

q ='nC7H16:1 O2:1 N2:3.76'

mech = 'SIP-Gr2.0-s2_mech.cti'

gas_initial = ct.Solution(mech)

gas_initial.TPX = T1, P1, q

rho_1 = gas_initial.density

# compute CJ speed

[cj_speed,R2,plot_data] = CJspeed(P1, T1, q, mech, fullOutput=True)

# compute equilibrium CJ state parameters

gas = PostShock_eq(cj_speed, P1, T1, q, mech)

ae = soundspeed_eq(gas)

af = soundspeed_fr(gas)

rho_2 = gas.density

gammae = ae**2*rho_2/gas.P

gammaf = af**2*rho_2/gas.P

w2 = cj_speed*rho_1/rho_2

u2 = cj_speed-w2

print ('CJ computation for ' + mech + ' with composition ' + q )

print ('Initial conditions: P1 = %.3e Pa & T1 = %.2f K'  % (P1,T1)  )

print ('CJ Speed   %.1f m/s' % cj_speed)

print (‘CJ State’)

print ('   Pressure   %.3e Pa' % gas.P)

print ('   Temperature  %.1f K' % gas.T)

print ('   Density  %.3f kg/m3' % gas.density)

print ('   Entropy  %.3e J/K' % gas.entropy_mass)

print ('   w2 (wave frame) %.1f m/s' % w2)

print ('   u2 (lab frame) %.1f m/s' % u2)

print ('   c2 frozen %.1f m/s' % af)

print ('   c2 equilibrium %.1f m/s' % ae)

print ('   gamma2 frozen %.3f ' % gammaf)

print ('   gamma2 equilibrium %.3f ' % gammae)

Output

CJ computation for SIP-Gr2.0-s2_mech.cti with composition nC7H16:1 O2:1 N2:3.76

Initial conditions: P1 = 1.013e+06 Pa & T1 = 1150.00 K

CJ Speed   1320.1 m/s

CJ State

Pressure   4.030e+06 Pa

Temperature  1508.5 K

Density  7.236 kg/m3

Entropy  1.062e+04 J/K

w2 (wave frame) 797.3 m/s

u2 (lab frame) 522.9 m/s

c2 frozen 810.1 m/s

c2 equilibrium 796.1 m/s

gamma2 frozen 1.178

gamma2 equilibrium 1.138